# Sample Flows

The following examples illustrate typical flows using the IPP protocol. In these examples, the IPP Printer object named “printer-1” is located at the node identified by the DNS name “some.domain.com”. A job Template has been defined for Printer-1 which establishes the print defaults.

For brevity in the following flows, none of the HTTP headers are shown. IPP headers are shown in bold type. CRLF sequences are not shown.

### Querying the printer

Client                                            some.domain.com

----------------------------------------------------------à

Post http://some.domain.com/printer-1   http/1.0

GetAttributes  IPP/1.0

Printer-state :

Sides-supported :

Media-supported :

Document-formats-supported :

ß----------------------------------------------------------

http/1.0  201 “Created” (a response)

IPP/1.0 xxx “attribute list returned”

Printer-state : running

Sides-supported : 1-sided

Media-supported : iso-a4-white, iso-b4-white

Document-formats-supported : Postscript/2.0

### Print Operation - with print data included

Client                                             some.domain.com

----------------------------------------------------------à

Post http://some.domain.com/printer-1  http/1.0

Print IPP/1.0

Print-Job-Object Header

Job-name : My Job

Medium : iso-a4-white

Notification-events : Job-completion

Notification-address : joe@pc.domain.com

Document Header

Document-name : Letter to Mom

Document-Content Header (content type = Postscript/2.0)

Document in Postscript level 2 format

ß-----------------------------------------------------------

http/1.0  200 “accepted”

IPP/1.0 xxx “print job accepted and queued”

Job-Identifier : some.domain.com/printer-1/0037

Current-job-state : pending

Printer-state : running

### Print Operation - with no data included

Client                                            some.domain.com

----------------------------------------------------------à

Post http://some.domain.com/printer-1  http/1.0

Print IPP/1.0

Print-Job-Object Header

Job-name : My Job

Medium : iso-a4-white

Notification-events : Job-completion

Notification-address : joe@some.domain.com

Document Header

Document-name : Letter to Mom

Document-URL : joe@pc.domain.com/Docs/To-mom.ps

ß-----------------------------------------------------------

http/1.0  200 “accepted”

IPP/1.0 xxx “print job accepted and queued”

Job-Identifier : some.domain.com/printer-1/0037

Current-job-state : pending

Printer-state : running

### Querying the state of the job - no attributes specified, so all job attributes are returned

Client                                            some.domain.com

----------------------------------------------------------à

Post http://some.domain.com/printer-1/0037  http/10,

GetAttributes  IPP/1.0

ß----------------------------------------------------------

http/1.0 201 “Created” (a response)

IPP/1.0  xxx “atribute list returned”

Job-Name : My Job

Job-Originator : Joe@some.domain.com

Job-originating-host : pc.domain.com

Notification-address : joe@pc.domain.com

Job-locale : xx:xx:xx

Current-job-status : printing

Printer-assigned : printer-1

Submission-time : 1214

Media-sheets-completed : 2

### Canceling a Job

Client                                           some.domain.com

---------------------------------------------------------à

Post: http://some.domain.com/printer-1/0037

CancelJob  IPP/1.0

ß---------------------------------------------------------

http/1.0  200 “okay”

Current-job-state : terminating

### Listing jobs on printer-1, only return job sizes. Jobs are returned in the order they are scheduled for printing. A Job-identifier attribute preceeds the attributes returned for each job to delimit job boundaries.

Client                                             some.domain.com

-----------------------------------------------------------à

Post http/1.0 some.domain.com/printer-1

GetJobs  IPP/1.0

total-job-octets :

ß-----------------------------------------------------------

http/1.0  201 “Created” (a response)

IPP/1.0 xxx “created an attribute list”

Job-identifier : 0033

total-job-octets : 4567

Job-identifier : 0034

total-job-octets : 12345

Job-identifier : 0035

total-job-octets : 12356