Proposed Internet-Draft	S. Isaacson

D. Taylor

Novell, Inc.

M. MacKay

Novell, Inc.

P. Zehler

Xerox Corporation

T. Hastings

Xerox Corporation

C. Manros

Xerox Corporation

R. Herriot

Sun Microsystems

November 11, 1996

Internet Printing Protocol

Version 0.905, November 11, 1996

NOTE: this document contains changes in the following sections:  Jobs 2.2.4 Job emplates, 4. Opeuding deletion of empty table rows), 6. User Operationson of empty table rows), 7. Object Attributes, to the end of

NOTE: Because this document has been extensively changed from its original form, it has many rough spots which will need further editing. At this time, the reader should read it for major concepts.

Status of this Memo

This document is a working version of a protocol specification.  It will eventually become an Internet-Draft by following well defined IETF procedures.  At that time, the following paragraphs must be included:

This document is an Internet-Draft.  Internet-Drafts are working documents of the Internet Engineering Task Force (IETF), its areas, and its working groups.  Note that other groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time.  It is inappropriate to use Internet-Drafts as reference material or to cite them other than as ``work in progress.''

To learn the current status of any Internet-Draft, please check the ``1id-abstracts.txt'' listing contained in the Internet-Drafts Shadow Directories on ftp.is.co.za (Africa), nic.nordu.net (Europe), munnari.oz.au (Pacific Rim), ds.internic.net (US East Coast), or ftp.isi.edu (US West Coast).

Abstract

This Internet-Draft specifies an Internet Printing Protocol (IPP) based on HTTP [still to be ratified by the IPP group].  protocol for the Internet. This protocol is heavily influence by  the semantic operations and attributes defined in ISO/IEC 10175 Document Printing Application (DPA) parts 1 and 3.  It also incorporates some of the implementation and interoperability lessons learned from other printing related standards such as POSIX System Administration - Part 4 (POSIX 1378.4) and X/Open A Printing System Interoperability Specification(PSIS).

IPP is defined as a set of abstract data types and operations. The operations are implemented using a protocol that is HTTP based [still to be ratified by the IPP group].

The IPP protocol initially covers only user operations on basic print service objects, but will cover management operation as soon as possible. Authentication and some access control will be required for the CancelJob operation. Additional access Control, Device Management, and Service Management will be added to the protocol as soon as possible. Some monitoring and management is possible through other protocols. The SNMP Printer MIB [1] is an example of one of these.  In the areas where there are no existing standards, many are being worked in other distributed service forums (management, security, etc.).  As these services become more standardized, this document (and hence the protocol) can be updated to reflect the integration and relationships with these other standards.

Table of Contents

1. Introduction	5

2.	Distributed Printing	6

2.1 Components	6

2.2 Objects	6

2.2.1 Printer	7

2.2.2 Job	9

2.2.3 Document	9

2.2.4 Initial Value Job	9

2.2.5 Initial Value Document	9

2.3 Object Relationships	9

2.4 Use of Naming and Directory Services	9

2.4.1 Status	10

2.4.2 Resolution	11

2.4.4 Color Supported	11

2.4.5 Maximum Speed	11

2.4.6 Maximum Speed Units	11

2.4.7 Plug and Play Device Id	11

2.4.8 Model	11

2.4.9 Manufacturer	11

2.4.10 Type	12

2.4.11 PDLs Supported	12

2.4.12 Sides Supported	12

2.5 OIDs	12

3.	Internet Printing Model	12

3.1 Object Instances 	12

3.2 Limits and Defaults	13

3.3 List Object Attribute Scoping Rules	13

4. Operations	13

4.1 Common Data Structures	13

4.1.1 XDR	13

4.1.2 ASN.1	16

4.2 Errors	17

4.2.1 ASN.1	17

5. Binding and Unbinding	19

5.1 Bind Operation	19

5.1.1 Bind Argument	19

5.1.2 Bind Result	21

5.2 Unbind Operation	22

5.2.1 Unbind Argument	22

5.2.2 Unbind Result	22

6. User Operations	22

6.1 Print Operation	22

6.1.1  Print Argument	22

6.1.2  Print Result	24

6.2 Cancel Job Operation	25

6.2.1  Cancel Job Argument	25

6.2.2  Cancel Job Result	26

6.3 List Object Attributes Operation	26

6.3.1  List Object Attributes Argument	27

6.3.2  List Object Attributes Result	30

6.4 Modify Job Operation	31

6.4.1  Modify Job Argument	31

6.4.2  Modify Job Result	32

6.5 Resubmit Job Operation	33

6.5.1 Resubmit Job Argument	33

6.5.2 Resubmit Job Result	34

7. Object Attributes	35

7.1 Job Attributes	36

7.1.1 Job Informational Attributes	37

7.1.2 Printer Selection Attributes	39

7.1.3 Job Status Attributes	39

7.1.4 Job Results Handling Attributes	43

7.1.5 Job Event Handling Attributes	44

7.1.6 Job Scheduling Instructions Attributes	45

7.2 Document Attributes	47

7.2.1 Document Description Attributes	47

7.2.2 Document Production Instruction Attributes	48

7.2.3 Document Characteristics Attributes	52

7.2.4 Document Status Attributes	54

7.3 Operation Attributes	54

7.4 Printer Attributes	54

7.4.1 printer-name	56

7.4.2 printer-state	56

7.4.3 message	56

7.4.4 printer-initial-value-job	56

7.4.5 printer-initial-value-document	57

7.4.6 fonts-supported	58

7.4.7 fonts-ready	58

7.4.8 media-supported	58

7.4.9 media-ready	58

7.4.10 printer-associated-printers	58

7.4.11 document-formats-supported	58

7.4.12 numbers-up-supported	59

7.4.13 finishings-supported	59

7.4.14 sides-supported	59

7.4.15 job-sheets-supported	59

7.4.16 document-sheets-supported	60

7.4.17 maximum-copies-supported	60

7.4.18 notification-delivery-methods-supported	61

7.4.19 physical-printers-supported	61

7.4.20 Logical-printers-supported	61

7.4.21 events-supported	61

7.4.22 transfer-methods-supported	61

7.4.23 locales-supported	61

7.4.24 multiple-documents-supported	61

7.4.28 cancel-individual-document-supported	62

7.4.29 modify-individual-document-supported	62

7.5 Initial Value Job Attributes	62

7.6 Initial Value Document Attributes	62

7.7 Relationship to ISO/IEC 10175 Conformance Levels	62

8.  Security Considerations	63

9.  References	63

10.  Author's Address	63

1. Introduction

This document is a Proposed Internet-Draft. It is a specification for a protocol that can be used for distributed printing on the Internet.  This protocol, Internet Printing Protocol (IPP)  is heavily influence by the printing model introduced in the Document Printing Application (ISO/IEC 10175 DPA) standard.  The DPA model describes a distributed printing service made up of cooperating networked entities.  DPA also identifies user and administrative roles and operations.  These ideas and concepts, when unified with other Internet protocols and services, realizes a distributed print service for the Internet.

This document assumes a distributed computing environment where print service users (clients, applications, drivers, etc.) cooperate and interact with print service providers (servers, printers, gateways, etc.) to realize the print service.

The actual protocol is yet HTTP based [still to be ratified by the IPP group].

2.	Distributed Printing

The distributed printing service is defined as a collection of coordinating and cooperating entities in a distributed computing environment. The model assumed by this protocol is potentially an n-tier client/server model, but the model will be optimized for the normal cases of a 1-tier model(client to printer) and a 2-tier (client to server to printer). Users of the 1-tier and 2-tier models should not be aware of any extra complexity to support 3 or more tiers A client is able to access the services offered by a server by invoking one or more operations associated with the server. Each operation has associated arguments and results.  The arguments provide additional data which is passed from the client to the server.  The results return the status and outcome of the desired operation back to the client from the server.

2.1 Components

In the distributed printing service the entities or components are:

-	One or more humans or agents acting on behalf of humans.  Humans (or their agents) act in the role of Users, Operators, Managers, or Administrators.

-	One or more clients.  Clients are computer network nodes with which end users interact in order to manipulate the distributed print service.  A client implements the IPP protocol.

-	One or more print service providers (servers).  An instance of a print service provider implements the IPP protocol by receiving and responding to IPP operations.  A print service provider is also a “client” of yet another instance of a print service provider. Some of these clients use the IPP protocol; others use some other protocol. The last print service provider in the n-tier chain is a “client” to a print engine. There are several different types of print service providers which are defined later in this document.

This LDPA specification only defines the operation used by Users.  The operations used by Operators, Managers, and Administrators may be added if there is time.

2.2 Objects

To accomplish the action(s) requested via an operation, the print service provider manages and manipulates data objects.  These are simply convenient collections of data that may represent other objects (real life or computer system) elsewhere.  A client supplies arguments in the form of attribute values for some of these objects.  A server informs the client of the status or outcome of an operation by also providing attribute values for the objects involved in the operations.  These objects are not encapsulations of both data and behavior as in other object oriented models, but are simple collections of attribute/value pairs. [We may try to fix this in our new design, but it’s not high priority.]

The objects which are relevant to this protocol are:

- Printer (contains server, queue and printer concepts)

- Job  (contains job and document concepts, a document object may

be added in the future)

- Job Template (contains Initial Value Job and Initial Value

Document)

IPP defines the operations that interact with and affect the real-life objects represented by the protocol’s object definition.

2.2.1 Printer

[Note: it is not clear when ‘Printer’ refers to the hardware and when it refers to a software printer object.]

This document shall use the following terms:

- 	Output Device: printer hardware,

- 	Print Server: a program that augments one or more Output Devices.

-	Printer: the software realization of a printer implemented in an Output Device or a Printer Server.

One of the most significant components within the distributed printing service is a Printer. A Printer object is a composition of some of the functionality that has traditionally been tied to other components within the printing system. A Printer can support the functionality of spooling, job management, device management, server, as well as more traditional device components.

A Printer can be in one of two authorization modes:

Public Access: The Printer is not restricted with any access control checks. The authorization allows anyone. The Printer uses a simple, name only (no password or credential) form of binding. [What does this last sentence mean?]

ISSUE: does Public access really mean no authentication. I

would expect not. Otherwise, a person can cancel anyone’s jobs.

Controlled Access: The Printer may have some restrictions based on some authentication and authorization scheme.  The Printer uses some form of credential based binding. . [What does this last sentence mean?]

A Printer object represents an instance of a print service provider which implements the IPP protocol.  This allows the Printer to provide a common interface for all types of disparate and diverse physical devices or as well as a gateway interface for other non-Internet based printing systems.

To a print service user, a Printer has the “looks and feel” of a any typical physical printer.   Jobs are submitted to and managed at the Printer.  The Printer can accept or reject submitted jobs based on job attributes which are sent along with the print job.  The Printer tracks all jobs that have been submitted to it.  The Printer can be modified to indicated a corresponding behavior change at the device level (either manually or automatically).  In the Controlled Access mode, the Printer has an identity with a security or credential service.

An object that an end-user views as a Printer can be implemented with either of the following configurations.

-	an Output Device which supports the IPP protocol. This Output Device may or may not have a job queue, and if it has a job queue it may be of very limited size. An administrator configures this Printer to receive jobs directly from a client.

-	a Print Server and one or more downstream Output Devices. The Print Server supports incoming IPP protocol and uses either IPP or some other protocol to communicate with downstream Output Devices. A Print Server augments downstream Output Devices by supporting a large job queue. An administrator configures the downstream Output Devices to receive jobs from the Print Server. When there is only one downstream Output Device, the Printer object in the Print Server has the same values as those in the downstream Output Device. Note that if the downstream Output Device supports the IPP protocol, then its Printer Object and the Print Server’s Printer Object are identical, attribute for attribute. When there are two or more downstream Output Devices, the Printer object in the Print Server has the union of values in the downstream Output Devices.

The print system shall also support gateways. Such gateways shall translate incoming IPP protocol to some other protocol and shall translate some other protocol to IPP protocol. Gateways shall pass operations through with minimal delay. Further description of them is beyond the scope of this document.

ISSUE: where does file conversion occur when there is no associated application, e.g. text or HTML to PostScript. To keep the model simple, the Printer defined above should only queue and print jobs. Conversion of text, HTML  should occur within some other server which forwards the job to a Printer.

The following describes the allowed chains of printer types using RFC 822 syntax:

normal usage:

ipp-end-user [PrintServer] OutputDevice

outbound gateway:

ipp-end-user Gateway AnotherPrintSystem

inbound gateway:

AnotherPrintSystem Gateway [PrintServer] OutputDevice

Although other arrangements of these printer types is possible, they are not useful.

2. How can I achieve both “fan out” and “fan in” of client to Printers and from Printers to physical print devices?

For “fan in”, an administrator or a user creates a name space entry which maps a name to a printer object and a job template.  This job template provides the default values to a client when a client creates a job object. In this model, the client does the work of defaulting, or alternatively, the server (possibly the Printer receiving a URL request) returns a HTML form with potential values and default values all filled in. The print service adds no additional default values for unspecified attributes, but an Output Device takes its own default action for unspecified attributes.

This model currently contains a mechanism for defaulting in two places: during GUI initialization in the client and during printing of the job in the Output Device. There may be a need to associated defaulting (i.e. a job template) with a Printer Object so that an administrator can enforce value that are left undefined.  This is currently an open issue that will not be resolved for version 1.0.

For “fan out”, an administrator sets up a Print Server to fan out to one or more downstream Output Devices  for load balancing and reliability.

3. How can a Printer be configured to service multiple spooling  Printers?

The system administrator establishes whether an end-user has direct access to an Output Device or indirect access via a Print Server. If multiple  Print Servers  are used to feed the same Output Device , their schedulers are not coordinated. Although this configuration is possible, it is not recommended.

2.2.2 Job

A Job object is used to model a job.  A job can consist of one or more documents.

There are no separate document objects, and no attributes that pertain to one document in a job but not to others, except for the attribute that specifies the document data, its location and format. .

In future versions, documents mayl become separate objects with attributes that override corresponding job attributes.

2.2.4 Job Template

An Job Template object is used to model job defaults.  A Job Template is  essentially a set of job attributes that a client uses to initialize a newly created job object.

2.3 Object Relationships

Instances of objects within the system have relationships which must be maintained persistently along with the persistent storage of the objects themselves.

An instance of a print service provider is a Printer.  The Printer is represented via a Printer object.  A Printer can contain zero, one, or more Job objects.  A Job object contains one or more Documents. The following relationships are examples:

•	“Document object D1 belongs to Job object J1”, or

•	“Job object J1 belongs to Printer object P1”.

2.4 Use of Naming and Directory Services

[ There is more work that need to be done to define the name service in an HTTP context. A name is a URL and is probably resolved at Print Servers and Output Devices.]

Any distributed service uses some sort of naming and/or directory service for (X.500, NDS, DCE naming, DNS).  It is outside the scope of this protocol to define which name service to use or what the protocol is for using that name service, but the following discussion helps to clarify how the name service is intended to be used.

To distributed printing system, the instances of print service providers are represented by objects of type Printer.  These same instances are also registered to the name/directory service.  There is one entry in the name service for each Printer.

That is, instances of print service providers are represented to IPP as Printer objects.  These objects represent their real-life counterparts, the print service provider (software, hardware, or firmware). However, for directory lookup, there is an entry in the naming service that also represents the Printer.

It is important to remember that a Printer object represents the current status and configuration information of a certain print service provider.  The Printer object contains attributes and values that describe the characteristics and capabilities of the print device.  However, a few of the most important attributes from the Printer object are duplicated in the entry in the directory.  These attributes are used for filtered directory lookups. The results of these searches enable a user to select an appropriate printer. It is the responsibility of the Printer itself to keep these attributes consistent and accurate.  This requirement frees the directory or some directory agent from continually polling registered entities for configuration changes.

The following attributes are in the directory entry:

Fully Distinguished Name (the name within the directory’s

name space)

Description

Location

Owner

Address

Status

Resolution

Color Supported

Maximum Speed

Maximum Speed Units

Device Id

Model

Manufacturer

Type

PDLs Supported

Sides Supported

The final set of name service entry attributes needs to be finalized and rationalized with the PSIS name service recommendations [6] as well as implementation experience.

2.4.1 Status

[ Such a dynamic value seem like it could be a problem in some name service entries.]

The printer status field in the directory entry is really a “summary” attribute of the true printer state.  The following mapping takes place between the Printer Status attribute in the directory entry and the printer-state attribute in the Printer object:

“Not Connected”

STATE_NOT_CONNECTED

STATE_PAUSED_NOT_CONNECTED

“Shutdown”

STATE_SHUTDOWN

“Active”

STATE_IDLE

STATE_PAUSED

STATE_PRINTING

“Stopped”

STATE_STOPPED

STATE_PAUSED_STOPPED

Even though the Printer  may not be up and running, the directory entry still exists in the directory.  In this case, the directory entry represents the fact that it may begin running at some future time.

2.4.2 Resolution

This is a single valued, maximum resolution in either the horizontal or vertical direction of the print device in dpi.

2.4.4 Color Supported

This is a BOOLEAN for either yes, color printing is supported, or no color printing is not supported.

2.4.5 Maximum Speed

This is the maximum speed of the printer in the units defined in Maximum Speed Units

2.4.6 Maximum Speed Units

This is the units of the maximum speed rating of the print device.  This can be: pages per minute, sheets per minutes, characters per second, etc.

2.4.7 Plug and Play Device Id

This attribute can be used for automatic driver download and other automatic configuration tasks.

2.4.8 Model

This is a simple text string defined by the manufacturer.

2.4.9 Manufacturer

This is a simple text string defined by the manufacturer.  There is no registration, and there is a possibility of overlap, but the goal is to keep this simple, not too complex.

2.4.10 Type

This is the printing mechanism of the print device: laser, ink jet, thermal, etc.

2.4.11 PDLs Supported

This is a list of all of the page description languages (PDLs) that the printer and/or its interpreter(s) support.

2.4.12 Sides Supported

This is either a 1 or a 2 to indicate the maximum number of sides on which the printer can automatically print.

2.5 OIDs

[OIDs do not belong in this model until we establish the protocol]

3.	Internet Printing Model

3.1 Object Instances

All instances of all objects have an identifier attribute that makes them unique so that they can be unambiguously referenced.  In the object-oriented model, these are the globally unique object references which are created by factories or constructors.

The following objects have the following mandatory identifier attributes:

Object				Identifier						Containing Object

Printer				printer-name					None

Job					job-identifier					Printer

Job Template		job-template-name				None

3.2 Limits and Defaults

This IPP specification does not include any mechanism for specifying for enforcing “limits” or any other kinds constraints.  However, defaults are achieved through the implementation of Job Template.

4. Operations

IPP defines the following end user operations:

The following symbols are used in the tables below:

P		perform the operation directly

PF		perform the operation; forward to Output Device sometimes

UA		unsupported in an Output Device unless it supports queuing

U		unsupported operation

| Operation | Print Server | Output Device |
| --- | --- | --- |
| - Print | PF | P |
| - Cancel Job | PF | P |
| - Get Attributes | PF | P |
| - Get Jobs | PF | P |

Lower priority (version 2) end user operations are:

| Operation | Print Server | Output Device |
| --- | --- | --- |
| - Modify Job | P | UA |
| - Resubmit Job | P | UA |

Management operations are (we want these in version 1.0):

| Operation | Print Server | Output Device |
| --- | --- | --- |
| - Clean Queue | PF | UA |
| - Disallow Queuing | P | UA |
| - Allow Queuing | P | UA |
| - Pause Printing | P | P |
| - Resume Printing | P | P |
| - Promote Job | PF | UA |
| - Shutdown Printer | P | P |
| - Startup Printer | P | P |
| - Create Printer | P | U |
| - Delete Printer | P | U |
| - Set Attribute | P | P |

4.1 Common Data Structures

This section describes the common data structures that are used by two or more operations.

6. User Operations

6.1 Print Operation

When an end-user uses GUI to submit a job, the GUI client gets an HTML form from the default printer.  If the end-user changes the selected printer, the GUI client gets the HTML form from that printer. The HTML form comes with the values supported by the printer and it is initialized by the values from the job template associated with the named printer.

[Further work needs to done to define the above concept.]

6.1.1  Print Argument

[We should be trying to create a protocol where the entire job is incorporated into a single transmission.  This eliminates the need for Add Document and Close Job.]

The following abstract data types are part of the Print Argument:

| Create Job | the Job Id is returned in the Print Results. |
| --- | --- |
| Printer Name |  |
| Job and Document Attributes |  |
| Attributes in Result | The names of attributes that the client wants in the result. Job-identifier is always returned. |
| All Document Contents |  |
| Common Arguments | Common to all three forms of Print Argument |

6.1.2  Print Result

The following abstract data types are part of the Print Result:

| Job Id | Used for all other operations on this Job. |
| --- | --- |
| Server State | Optional state information about the Print Service Provider |
| Message | Optional message |
| Job Status | Job state information |
| Errors | Optional Error Information |

6.2 Cancel Job Operation

6.2.1  Cancel Job Argument

The following abstract data types are part of the Cancel Job Argument:

| Job Id | The identifier of the job to be canceled. |
| --- | --- |
| Document Number | Optional document number of the document to cancel within a given job. [ probably not supported] |
| Message | Optional message to the operator. |
| Retention Period | Optional period for retaining the cancelled job. |
| Common Arguments |  |

Result

The following abstract data types are part of the Cancel Job Result:

| Job Status | Optional Job status information |
| --- | --- |
| Errors | Optional Error Information |

6.3 Get

6.3.1  Get  Attributes Argument

The following abstract data types are part of the Get Attributes  Argument:

| SPECIFICATION |  |
| --- | --- |
| Selector | A job or printer name URL (the class is implicit in the object named) |
| Requested Attributes | A set of attributes in which the requestor is interested |
|  | [ I would like to get rid of this if possible] |
| Common Arguments |  |

6.3.1  Get Jobs Argument

The following abstract data types are part of the Get Jobs  Argument:

| Selector | A printer name |
| --- | --- |
| Filtering | A lightweight filtering mechanism, such as all jobs versus a particular user’s jobs. |
| Requested Attributes | A set of job attributes in which the requestor is interested |
| Common Arguments |  |

6.3.2  Get Attributes and Get Jobs Result

The following abstract data types are part of the List Object Attributes Result:

| Result Attributes | Attribute set containing the returned results. |
| --- | --- |
| Errors | Optional Error Information |

6.4 Modify Job Operation

6.4.1  Modify Job Argument

The following abstract data types are part of the Modify Job Argument (the attributes that can be modified may be severely restricted):

| Job Id | Which job to modify. |
| --- | --- |
|  | [There are no document attributes to modify.] |
| Job Attributes | Attribute set for Job attributes. Only replacement is possible; the GUI fetches the value and then sets a new one. |
| Message | Optional Message. |
| Common Arguments |  |

6.4.2  Modify Job Result

The following abstract data types are part of the Modify Job Result:

| Modify Status | Modify result attributes. |
| --- | --- |
| Errors | Optional Error Information |

6.5 Resubmit Job Operation

6.5.1 Resubmit Job Argument

The following abstract data types are part of the Resubmit Argument:

|  | . |
| --- | --- |
| Destination Printer Name | Optional name of the destination printer. |
| Operation | MOVE or COPY |
| Job Set | A set of jobs to move or copy. Each entry in the set has: Job Id, Document Number, Job attributes, and Document attributes. |
| Message | Optional Message |
| Common Arguments |  |

6.5.2 Resubmit Job Result

The following abstract data types are part of the Resubmit Job Result:

| Resubmit Job Set | A set of jobs that were resubmitted. Each element in the set has: Old Job Id, New Job Id, and an attribute set with info about the results of the move or copy. |
| --- | --- |
| Errors | Optional Error Information |

7. Object Attributes

This section describes the attributes and their associated values that are part of the IPP protocol. The list below shows the objects and their attributes that are included within the scope of this protocol:

In the following sections, the text has been heavily influenced by the  ISO/IEC 10175 DPA (Final, June 1996).

7.1 Job Attributes

A job object contains a set of job attributes and one or more documents.  A client shall create a job and send it to a server using the Print operation. -A client may use a job template associated with the selected printer in order to initialize the job.

Each section heading below contains the name of an attribute and its syntax in parentheses using the rules of RFC 822.

The sections below reference the following syntax items:

string: TBD

stringPair: string “:” string

stringState: string state

name: TBD

URL: TBD

dateTime: TBD

deltaTime: [hours “:”] minutes

cardinal: TBD

type1Enum: TBD

type2Enum: TBD

type3Enum: TBD

type2EnumState: type2Enum state

type3EnumState: type3Enum state

state: TBD

Boolean: TBD

positiveInteger: TBD

positiveIntegerCross: positiveInteger [ “x” positiveInteger ]

positiveIntegerCrossState: positiveIntegerCross state

positiveIntegerRange: positiveInteger  “:” positiveInteger

positiveIntegerUnits: positiveInteger units

positiveIntegerState: positiveInteger state

units: “ppm” | “ipm” | “spm” | “cps” | “lpm”

type3Locale: type3Country “:” type3Language “:” type3CodeSet

type3Country: type3Enum

type3Language: type3Enum

type3CodeSet: type3Enum

type2Format: name [ “/” version ]

version: name

type3LocaleState: type3Locale state

7.1.1 Job Informational Attributes Set by a Client

The client may specify these attributes in the Print operation to provide information to identify a print-job.

The client may also specify these attributes in the operations: Get-Attributes, and Get-Jobs.

7.1.1.4 job-name (string)

This attribute supplies a human readable string for naming the print-job.

This attribute is intended for to be printed on a start sheet, returned in a Get-Jobs result, or used in notification messages.

If the client does not specify this attribute, a Printer shall set it to the name of the file of the first document in the job.

7.1.1 Job Informational Attributes Set by a Printer

The Print shall add all of these attributes to a job to provide information to identify a print-job.

The client may specify these attributes in the operations: Get-Attributes and Get-Jobs, but not in Print.

-

---

------

7.1.1.1 job-identifier (string)

This attribute provides the job-identifier for this job on the Printer.  The Printer  shall generate a job-identifier value that is unique on that Printer, but need not be unique across the distributed environment.

The value of the job-identifier attribute shall be returned by the Printer  as part of the PrintResult in the Print -operation.  -

7.1.1.2 job-originator (name)

This attribute specifies the name of the person submitting the print job.  The Printer shall set this attribute to the most authentic name that it can obtain from the client. The operation-user-name attribute is intended to be a source of the most authentic name.

7.1.1.2 job-originating-host (name)

This attribute identifies the originating host of the job. The Printer shall set this attribute to the value of the operation-host-name which is intended to be the most authentic host name of the client.

7.1.5.1 notification-address (name)

This address specifies the email address of the client. The client specifies this attribute in the operation-attribute which the Printer in turn uses to set this attribute.

The Printer shall use this attribute as the address for sending messages to a job submitter when an event occurs that the user has registered an interest in or when certain other events occur, such as Cancel-Job.

Note: The only type of notification is email.

ISSUE: can the email address be inferred with job-originator and the originating-host.

7.1.1.2 job-locale (type3Locale)

This attribute identifies the locale of the job. The Printer sets this attribute from the value of the operation-locale.

The Printer shall use this attribute to determine the locale for notification messages that it sends.

The type3EnumTrip consists of 3 colon separated type 3 enums. The first is the country. The second is the language. The third is the code-set.

ISSUE: is there a more standard syntax for locale?

----

7.1.2 Printer Selection Attributes

The client shall specify this attribute to select a particular Printer. -

The client may also specify these attributes in the operations: Get-Attributes, and Get-Jobs.

Issue: this attribute may be implicit in the specified URL in the Print operation.

-

--

--

7.1.2.1 printer-name-requested (URL)

This attribute identifies the printer that the client requests  for printing the job.

7.1.3 Job Status Attributes

The Printer shall add these attributes to a job when a client submits a job, and the Printer shall assign appropriate values to each such job-status attribute.

The Printer uses these attributes to specify the job status before, during and after the processing of the print-job by the Print. -

The client may specify job-status attributes in: Get-Attributes and Get-Jobs, but not Print.

7.1.3.1 current-job-state (type1Enum)

This attribute identifies the current state of the job with the following values: unknown, pre-processing, pending, processing, printing, held, terminating, retained, completed.

---------------------------------

The IPP protocol supports all values for job states, but Printers are need  only support those states which are appropriate for the particular implementation.

7.1.3.2 printer-assigned (name)

This attribute identifies the Output Device to which the Printer has assigned this job.

If an Output Device implements a Printer, the Printer does not set this attribute.

If a Print Server implements a Printer, the value shall be empty until the Printer assigns an Output Device to the job..

ISSUE: Is this attribute appropriate for a model in which we are hiding the downstream Printer.

The printers-assigned value shall not be the same as the printer requested by the user.

The value of the job's printer-assigned attribute shall remain after the job has completed, so that users can determine the Output Device on which the job was printed.

7.1.3.3 submission-time (dateTime)

This attribute indicates the time at which the this job was accepted by the Printer.  If the Printer does not support the notion of time, the attribute is not stored as part of the job object.

7.1.3.5 job-message-from-administrator (string)

This attribute provides a message from an operator, system administrator or ‘intelligent' process to indicate to the user the reasons for modification or other management action taken on a job.

7.1.3.6 completion-time (dateTime)

This attribute indicates the time at which this job completed. This time is useful for jobs which are retained after printing.

7.1.3.7 job-state-reasons (1#type2Enum)

This attribute identifies the reason or reasons that the job is in the state that it is in (e.g., held, terminating, retained, completed, etc.).  The printer shall indicate the particular reason(s) by setting the value of the job-state-reasons attribute.  It is valid for the printer to set the value of the job-state-reasons attribute to the empty set.

The following standard values are defined: ---documents-needed, ---job-hold-set, ---job-print-after-specified, job-off-peak-specified, ---required-resources-not-ready, ---successful completion, ---completed-with-warnings, ---completed-with-errors, ---cancelled-by-user, ---cancelled-by-operator, ---aborted-by-system, ---logfile-pending, ---logfile-transferring

| -- |  |  |  |
| --- | --- | --- | --- |

7.1.3.9 impressions-completed (cardinal)

This attribute contains the number of impressions that the Printer has completed printing.  If the Printer cannot report this number, the Printer leaves this attribute unspecified.

7.1.3.9 media-sheets-completed (cardinal)

This attribute contains the number of media-sheets that the Printer has completed printing.  If the Printer cannot report this number, the Printer leaves this attribute unspecified.

7.1.4 Job Sheet  Attributes

The client shall specify these attributes to control the printing of of job sheets.

The client may also specify job sheet attributes in: Get-Attributes and Get-Jobs.

--

7.1.4.1 job-sheets (type3Enum)

This attribute determines what type of job-sheets the Printer  shall print with the job.

The standard  values are: none, and default-sheet.

The value ‘None’ means that the Printer shall print no job sheets.  The value ‘Default’ means that the Printer shall print the job sheets defined by an administrator.

7.1.5 Notification  Attributes

The client shall specify these attributes to indicate events that the client is interested in.

The client may also specify notification attributes in: Get-Attributes and Get-Jobs.

7.1.5.1 notification-events (#type1Enum)

This attribute specifies the  events about which the user want  to be notified.

This attribute will support three events classes: job-completion, job-problems and printer-problems., If attribute contains no values, then the client is requesting no notification.

This attribute will support only one delivery method, namely email. The attribute notification-address specifies the email address.

If this attribute contains the event job-completion, the Printer shall notify the client when the job containing this attribute completes.

If this attribute contains the event job-problem, the Printer shall notify the client when the job containing this attribute has a problem while the job is printing. Problems include: paper jam and out-of-paper.

If this attribute contains the event printer-problem, the Printer shall notify the client when the job containing this attribute has a problem while the job is printing or waiting to print. Problems include: paper jam and out-of-paper.

ISSUE: is there a problem with an attribute with an empty value being different from no attribute. Otherwise, we need a special value of ‘none’. ‘none’, unlike other values should not be combined with other values.  This particular case does not seem like a case where an administrator wants to prevent the use of ‘none’, so empty seems like a good solution.

ISSUE: Email is quite deficient for timely notification to an end-user who receives a lot of email, but there are no other choices.  The internet community needs to solve this problem, perhaps with an extremely-urgent email.

7.1.6 Job Scheduling Attributes

The client shall specify these attributes to provide the Printer with information  for the scheduling a print-job. --

The client may also specify these -- attributes in: Get-Attributes and Get-Jobs.

7.1.6.1 job-hold (Boolean)

This attribute specifies whether the print-job is a candidate for scheduling for printing or not, when the Printer  would otherwise place the job in the pending or processing states

When the value is FALSE, the Printer shall not hold the job from being scheduled for printing, unless there are other reasons (see the current-job-state and the job-state-reasons job-status attributes).

When the value is TRUE, the Printer shall place the job in the held state and add the job-hold-set value to the job's job-state-reasons attribute and shall not schedule the print-job for printing.  If the job enters the held state because its job-hold attribute was TRUE, a client shall reset the job's job-hold attribute to FALSE by means of the ModifyJob operation before the printer can schedule the job for printing.  When the value is set to FALSE as a result of the ModifyJob operation, the printer shall remove the job-hold-set value from the job-state-reasons attribute and, if no other reasons remain, shall change the job's current-job-state to pending so that the job becomes a candidate for being scheduled on printer(s).

ISSUE: the above few sentences assume a ModifyJob operation, which is not in version 1.0. But without this operation, the job-hold operation is not very useful. Perhaps we should remove job-hold.

7.1.6.2 job-priority (type1Enum)

This attribute specifies a priority for scheduling the print-job. Printers  that employ a priority-based scheduling algorithm use this attribute.

There are three standard values: high, medium, and low. Among those jobs that are ready to print, a Printer shall print all such jobs with a high priority before printing those with a medium or low priority, and a Printer shall print all such jobs with a medium priority before printing those with a low priority. ---

If the client does not specify this attribute, the Printer assumes that the user places no constraints concerning priority on the scheduling of the print-job, and it has a priority value of the value of the Printer’s maximum-end-user-priority attribute. If that attribute is unspecified, the Printer uses the value of ‘high’.

An operator can modify a job to have any priority. An end-user is restricted to the value of printer attribute maximum-end-user-priority.

7.1.6.3 job-print-after (dateTime)

This attribute specifies the calendar date and time of day after which the print-job shall become a candidate for printing.

If the value of this attribute is in the future, the Printer shall set the value of the job’s current-job-state to held and add the job-print-after-specified value to the job’s job-state-reasons attribute and shall not schedule the print-job for printing until the specified date and time has passed.  When the specified date and time arrives, the Printer shall remove the job-print-after-specified value from the job’s job-state-reason attribute and, if no other reasons remain, shall change the job’s current-job-state to pending so that the job becomes a candidate for being scheduled to print.

If this attribute is unspecified or the value is in the past, the job shall be a candidate for scheduling immediately.

7.1.6.3 job-print-off-peak (type3Enum)

This attribute specifies the off-peak period during which the print-job shall become a candidate for printing.

If this attribute is specified, it contains a value with which an administrator has associated allowable print times. An administrator is encouraged to pick names that suggest the type of off-peak period, such as ‘night’, ‘weekend’, ‘thirdShift’.

If this attribute is unspecified, the job shall be a candidate for scheduling immediately.

7.1.6.4 job-retention-period (deltaTime)

The retention time is expressed in hours and minutes, e.g. 6:00 (6 hours), or 20 (20 minutes).

This attribute specifies the minimum period of time following the completion of job processing and printing that the server shall keep job attributes and document data.  The Printer may keep these attributes and data longer than the value of the job-retention-period attribute.

----

----

--

-

-

| - |  |  |  |
| --- | --- | --- | --- |

7.2.2 Job Production Attributes

The client shall specify these attributes to affect the rendering, production and finishing of the document..  Similar types of instructions  may also be contained in the document to be printed.

If there is a conflict between the value of one of these attributes, and a corresponding instruction in the document (either implicit or explicit), the value of the attribute shall take precedence over the document instruction.

After the information from these attributes has been folded into the document data (possibly during a translation process of the document data), these attributes  are no longer relevant and can be discarded from a job.  Instead, the resource attributes specify  the resources needed to print the job as modifieed by the job production attributes.

Note: until companies that supply interpreters for PDL’s, such as PostScript and PCL allow a way to specify overrides for internal job production instructions, a Printer may not be able to implement these attributes for some PDL’s.

The client may also specify document production-instruction attributes in: Get-Attributes and GetJobs.

7.2.2.2 medium-select (type2Enum)

This attribute identifies the medium that the Printer shall use for all  pages of the document regardless of what media are specified within the document.

The values for medium include medium-names, medium-sizes, input-trays and electronic forms so that one attribute specifies the media.

Standard values are defined: TBD

7.2.2.3 number-up (positiveInteger)

This attribute specifies the number of source page-images to impose upon a single instance of a selected medium. -.

In general, only certain numeric values are valid for this attribute, depending upon the Printer implementation to which the print-request is directed. Typical supported values are 2 and 4.

If this attribute is unspecified or has a value of 1, then the Printer does not apply any number-up transformation to the pages.

This attribute primarily controls the translation, scaling and rotation of page images, but a site may choose to add embellishments, such as borders to each logical page.

ISSUE: should there be a separate attribute to control embellishments, especially for the 1-up case.

------------------

--

-------------------------

7.2.2.4 finishing (type2Enum)

This attribute identifies the finishing operation that the Printer should apply to -each copy of the printed document.

- Examples include stapling, saddle-stitching, hole-drilling, binding with tape, etc.

Standard values for this attribute are include:TBD.

7.2.2.5 sides (type2Enum)

This attribute specifies whether the document should be printed in one of three ways: 1-sided (simplex), 2-sided- long-edge (duplex), 2-sided- short-edge (tumble).

The standard values are: 1-sided, 2-sided-long-edge, 2-sided-short-edge.

7.2.2.6 copies  (positiveInteger)

This attribute specifies the number of copies of the job to be printed. If this attribute is unspecified, its default value is 1 copy.

7.2.2.1 printer-resolution-select (positiveIntegerCross)

This attribute specifies the resolution that the Printer should use.

The syntax allows a single integer to specify the resolution or a pair of integers to specify the resolution when the x and y dimensions differ. When two integers are specified, the first is in the paper feed direction.

7.2.2.1 print-quality (type2Enum)

This attribute specifies the print quality that the Printer should use.

The standard values are: TBD.

7.2.2.1 page-select (positiveIntegerRange)

This attribute specifies the pages in the document that the Printer shall  use. This attribute is unlikely to be useful for jobs with more than one document or in Job Templates. If this attribute is unspecified, then the Printer prints all pages in a document.

7.2.2.1 files-are-one-document (Boolean)

This attribute is relevant only if a job consists of two or more documents. It controls finishing operations, job-sheet placement, and the order of documents when the copies attribute exceeds 1.

If the files for the job are a and b  and this attribute is true, then files a and b are treated as a single document for finishing operations. Also, there will be no slip sheets between files a and b. If more than one copy is made, the ordering must be  a, b, a, b, …. The attribute files-are-interleaved is ignored.

If the files for the job are a and b  and this attribute is false or unspecified, then each file is treated as a single document for finishing operations. Also, a client may specify that a slip sheet be between files a and b. If more than one copy is made, and the attribute files-are-interleaved false or unspecified, the ordering is a, a, b, b, …. If more than one copy is made, and the attribute files-are-interleaved true, the ordering is a, b, a, b, ….

7.2.2.1 files-are-interleaved (Boolean)

This attribute is used in conjunction with files-are-one-document (q.v.).

7.2.2 Attributes for Conversion of Text Files

The client shall specify these attributes to control  formatting for text documents or HTML documents. If the client does not specify any of these attributes, a Printer shall uses its own defaults.

A client need not specify these attributes for other types of documents, such as PostScript or PCL.

7.2.2.1 width (cardinalUnits)

This attribute specifies the media width for the document.

7.2.2.1 length (cardinalUnits)

This attribute specifies the media length for the document.

7.2.2.1 left-margin (cardinalUnits)

This attribute specifies the left-margin for the document.

7.2.2.1 right-margin (cardinalUnits)

This attribute specifies the right-margin for the document.

7.2.2.1 top-margin (cardinalUnits)

This attribute specifies the top-margin for the document.

7.2.2.1 bottom-margin (cardinalUnits)

This attribute specifies the bottom-margin for the document.

7.2.2.1 repeated-tab-stops (cardinalUnits)

This attribute specifies the tab stops for the document.

7.2.2.1 header-text (string)

This attribute specifies the header text for the document.

7.2.2.1 footer-text (string)

This attribute specifies the footer text for the document.

7.2.2.1 number-pages (Boolean)

This attribute specifies that the pages should be numbered in the document.

7.2.2.1 default-font (string)

This attribute specifies the font to use for all text in the document.

7.2.2.1 default-code-set (type3Enum)

This attribute specifies the code-set in which the document is encoded.

7.2.2.1 content-orientation (type2Enum)

This attribute specifies the orientation of the document.

The standard values are: landscape or portrait.

7.2.3 Job Resource Attributes

A program described below shall add these attributes, which describe the resources needed to print the job. --

A Printer  may use these attributes to validate and schedule the print-job without interpreting the contents of the document. This provides the opportunity for a Printer to support a broad set of document formats yet still support fast efficient scheduling and validation of each job.

The client shall not specify these attributes.  Instead, it is the duty of the program  that translates the document to the printer’s PDL (or analyzes it) to add these attributess and their values to the job. If any of these attributes is unspecified, the Printer shall assume that the all resources required by the document of the type specified by the missing attributes are ready. These attributes may be unspecified if the translation program fails to provides such values, or if no translation occurs (e.g. the document is a PostScript document.

Note: The Printer does not use these attributes during the actual printing of a document.

Note: these attributes allow more than one value wherever it is possible for a job to specify more than one value of the corresponding job attribute, possibly by embedded instructions.

The client may specify these - attributes in: Get-Attributes and Get-Jobs.

7.2.3.1 document-format-used (1#type2Format)

This attribute identifies the document format needed to print this job.

-----

A format  consists of two elements, a name and a version. The latter element is optional.

The syntax is for type2Format:

name [ “/” version ]

Examples include: PostScript, PostScript/2.0 and PCL/5e

ISSUE: do we want the version to be optional?

X

-----

----

-----

-----

7.2.3.1 fonts-used (1#string)

This attribute identifies the font resources used  in the job.

7.2.3.1 code-sets-used (1#type3Enum)

This attribute identifies the code-sets used in the document. This attribute is relevant only for files that are not in ASCII, such as text files and possibly PCL files. PostScript files are always ASCII. Normally there is at most 1 code-set.

Standard values are defined in the section on default-fonts.

7.2.3.2 media-used (1#type2Enum)

This attribute identifies the media, media-sizes, input-trays or electronic forms needed to print the job.

Standard values for this attribute are defined in the section on medium-select.

--

7.2.3.2 sides-used (type2Enum)

This attribute specifies whether a job needs one-sided, two-sided-long-edge, or two-sided-short-edge printing.

Standard values for this attribute are defined in the section on sides.

7.2.3.2 print-quality-used (type2Enum)

This attribute specifies what print quality the job needs.

Standard values for this attribute are defined in the section on print-quality.

7.2.3.2 finishing-used (type2Enum)

This attribute specifies what finishing the job needs.

Standard values for this attribute are defined in the section on finishing.

7.2.3.2 printer-resolution-used (positiveIntegerCrossState)

This attribute specifies what resolution the job needs.

7.2.3.2 total-job-octets (positiveInteger)

This attribute specifies the total size of the job in octets. This attribute is the first of three that a translation program can use to specify the size of a job.

7.2.3.2 job-impression-count (positiveInteger)

This attribute specifies the total size of the job in impressions.

7.2.3.2 job-media-sheet-count (positiveInteger)

This attribute specifies the total size of the job in media-sheets.

7.2.4 Number of Documents

This group contains a single attribute which specifies the number of documents in the job.

The client shall specify this attribute in Print and may specify this attribute in: Get-Attributes and Get-Jobs.

7.2.1.1 number-of-documents (positiveInteger)

This attribute specifies the number of documents in the job. Each document shall contain its own set of document content attributes described below.

7.2.4 Document Data

This group of attributes describes the document data for the job. These attributes also include the document data or reference it.

All job attributes in other sections of this document occur only once per job and apply to all documents in a job.

The client may specify document-data attributes in Print. The client must specify either the document-URL or document-path in Print.

Except for document-content, the client may specify document-data attributes in: Get-Attributes, and Get-Jobs.

-

----

---

7.2.3.1 document-format (type2Format)

This attribute identifies the document format of this document.

If the client does not specify this attribute, then the Printer shall attempt to determine the format in order to decide if the document data needs to be translated.

ISSUE: do we want the version to be optional?

7.2.1.2 document-name (name)

This attribute contains the name of the document used by the client to initially identify the document.

7.2.1.2 document-URL (name)

This attribute contains the URL of the document if the client specified the document with a URL.

If this attribute is specified, then document-content and document-path shall be unspecified.

7.2.1.2 document-content (octetString)

This attribute contains the actual contents of the document.

If this attribute is specified, then document-path and document-URL shall be unspecified.

This attribute shall be used during the transmission of the Print operation over a network. A Printer shall save the document data to a file and reference it with the document-URL or document-path attribute. A Get-Attribute or Get-Jobs operation shall always find that this attribute is unspecified.

7.2.1.2 document-path (name)

This attribute contains a path which references a file containing the document.

If this attribute is specified, then document-content and document-URL shall be unspecified.

This attribute shall not be used during the transmission of the Print operation over a network. It is intended to reference the file when document data is on the printer.

ISSUE: is this attribute necessary or is document-URL sufficient?

7.3 Operation Attributes

The client shall set these attributes and associate them with an operation rather than an object.

It is intended that a client program rather than an end-user has control over the setting of these values so that they cannot be easily forged.

7.4.1 operation-locale (type3Locale)

This attribute identifies the locale of the client. The Printer uses this attribute to determine the locale of messages in the result of the operation or in errors returned by the operation.

The type3EnumTrip consists of 3 colon separated type 3 enums. The first is the country. The second is the language. The third is the code-set.

If an operation does not specify this attribute, the Printer shall assume that the operation has the same locale as the Printer.

7.4.1 operation-notification-address (name)

This attribute identifies the email-address of the client. The Printer uses this attribute to determine the email address for any notifications that occur in the Printer.

ISSUE: can this address be determined from the next two attributes: operation-user-name and operation-host-name?

7.4.1 operation-user-name (name)

This attribute identifies the most authenticated user name that the client can supply. This name identifies the user performing the operation.

This value shall be set by the system rather than the end-user in order to minimize the chance of forgery.

7.4.1 operation-host-name (name)

This attribute identifies the most authenticated host name that the client can supply. This name identifies the host from which the operation comes.

This value shall be set by the system rather than the end-user in order to minimize the chance of forgery.

7.4 Printer Attributes

A printer object may be realized in  either a Print Server or Output Device .

A Printer Object in an Output Device  contains a set of printer object attributes that represent an Output Device capable of rendering a document in visible form.  Examples include electronic and electro-mechanical printers such as laser printers, ink-jet printers, and various kinds of impact printers, but may include other types of output devices such as microfiche imagers and plotters as well.

A Printer Object in a Print Server  contains a set of printer object attributes that are the union of the Printer objects in the downstream Output Devices.  This object extends the capabilities of an Output Device.  For example, an administrator might define a single Print Server to represent all of the Output Devices of the same type and capability in a single location, associated with a particular server.  A user/client would normally send a print-job to a Print Server , and allow the Print Server to assign the job to a particular Output Device  based on the relative load and availability of the printers under its control, thus providing a load balancing service.  However, nothing  precludes an administrator from configuring a print system so that a user/client can send a print-job directly to an Output Device .

A Print Server, in the most common case, controls exactly one downstream Output Device. The Print Server’s Printer object has attributes whose values are the same as those of the Printer object in the downstream Output Device.

The attributes defined in this section  provide information about a particular Printer.

7.4.1 printer-name (name)

This attribute uniquely identifies the printer on its host.

7.4.1 printer-location text (string)

This attribute identifies the location of this printer.

7.4.1 printer-model (string)

This attribute identifies the make and model of the printer.

7.4.1 printer-types (type2Enum)

This attribute identifies the marking technology of the printer.

The value for this attribute are the descriptive names specified by ISO DPA. These values are: other, unknown. electrophotographic-LED, electrophotographic-laser, electrophotographic-other, impact-moving-head-dot-matrix-9-pin,  impact-moving-head-dot-matrix-24-pin, impact-moving-head, dot-matrix-other, impact-moving-head-fully-formed, impact-band, impact-other, inkjet-aqueous, inkjet-solid, inkjet-other, pen, thermal-transfer, thermal-sensitive, thermal-diffusion, thermal-other, electro, erosion, electro-static, photographic-microfiche, photographic-imagesetter, photographic-other, ion-deposition, E-beam, typesetter.

ISSUE: Should they be from the printer MIB instead. In the printer MIB hyphens do not exist. Instead the first letter after a hyphen is upper case.

7.4.2 printer-state (type1Enum)

This attribute identifies the current state of the printer.  The protocol support all values for printer states, however a Printer shall only generate the printer states which are appropriate for the particular implementation.

The following standard values are defined: ----unknown, ----idle, ----printing, ----needs-attention, ----paused, ----shutdown, ----job-start-wait, ----job-end-wait, ----job-password-wait, ----needs-key-operator,

----connecting-to-printer,

---state-timed-out

7.4.2 printer-state-message (string)

This attributes specifies a message that gives further information about the current printer state. .

7.4.3 message (string)

This attribute provides a message from an operator, system administrator or ‘intelligent' process to indicate to the user the reasons for modification or other management action taken on a job.

7.4.6 locale (type3Locale)

This attribute specifies the locale that the Printer operates in.

The standard values are defined in the section on the job-locale attribute.

7.4.6 notification-events (#type2Enum)

This attribute specifies the events on whose occurrence the Printer should notify those addresses specified by the notification-addresses attribute.

If the attribute is unspecified or empty, the Printer does not perform notification, though the Printer still checks the jobs’ notification-events attribute.

In this attribute, job-problem and printer-problem have the same meaning.

The standard values are defined in the section on the job’s notification-events attribute.

7.4.6 notification-addresses (#name)

This attribute specifies the email addresses to which the Printer should send messages when events specified by the notification- events attribute occur.

If the attribute is unspecified or empty, the Printer does not perform notification, though the Printer still checks the jobs’ notification-events attribute.

7.4.6 end-user-acl (#name)

This attribute specifies the users who are allowed to print on the Printer.

If the attribute is unspecified or empty, the Printer allows anyone to print.

ISSUE: this does not fully solve the internet authorization problem because of authentication issues.

7.4.15 maximum-printer-speed (positiveIntegerUnits)

This attribute indicates the maximum printer speed of the Printer. A job cannot control a Printer’s speed, but a Printer Browser can use printer speed as a criteria.

The standard units are a type2Enum and are: ppm, ipm, spm, lpm, cps.

7.4.6 fonts-substitutions (#stringPair)

This attribute specifies an appropriate substitute for a font that is advertised as supported in the fonts-supported attribute, even though the Printer doesn’t actually have the font available.

This attribute consists of a set of font pairs: a font name and the font to use instead.

7.4.6 fonts-supported (1#stringState)

This attribute identifies the font resources supported by this printer and indicates the state of readiness for each font.

The standard names are defined in the section on default-font.

Each item in the list contains the pair consisting of a font name and a state indicating the font’s readiness state.

7.4.8 media-supported (1#nameState)

This attribute identifies the media, media-sizes, input trays, and electronic forms supported by this printer, and indicates the state of readiness for each medium resource.

There may be just two states: ready and needs-installing, or there may be a third state: needs-purchasing.

The standard names are defined in the section on the section on the medium-select.

7.4.11 document-formats-supported (1#type2FormatState)

This attribute identifies the document-formats, including the --document-format-versions, supported by the Printer. This set includes both the formats that are native to the Printer  and those formats that the Printer  can translate to one that is native to the Printer. From the client's point of view, this set contains all formats in which documents can be submitted to this Printer.

Proprietary document format identifiers, and versions are assigned by the owners of those formats.

The state of readiness for each format is also included, though all formats should normally always be ready.

7.4.12 numbers-up-supported (1#positiveIntegerState)

This attribute identifies the number-up values supported by this printer.--.

The state of readiness for each number-up value is also included, though all number-up conversions should always be ready.

7.4.13 finishings-supported (#type2EnumState)

This attribute identifies the per-document finishing operations  supported by this Printer and states of readiness for each finishing.-

The standard finishing objects are defined in the section on the finishing attribute.

7.4.14 sides-supported (1#type2EnumState)

This attribute indicates the values of the sides attribute supported by this printer and the states of readiness of each value.

The standard values are defined in the section on the sides attribute.

7.4.15 print-qualities-supported (1#type2EnumState)

This attribute indicates the values of the printer-quality attribute supported by this printer and the states of readiness for each print-quality value.

The standard values are defined in the printer-quality attribute.

7.4.15 printer-resolutions-supported (1#positiveIntegerCrossState)

This attribute indicates the values of the printer-resolution-select attribute supported by this printer and their states of readiness.

The state of readiness for each printer resolution is also included, though normally all printer-resolutions should always be ready.

The syntax is discussed in the section on the printer-resolution-select attribute.

7.4.15 code-sets-supported (1#type3EnumState)

This attribute indicates the values of the default-code-set attribute supported by this printer and the states of readiness for each code-set.

The standard values are defined in the default-code-set attribute.

7.4.17 off-peak-times-supported (#type3EnumState)

This attribute indicates the values of the job-print-off-peak attribute supported by this printer and the states of readiness for each value.

If this attribute is unspecified, then the Printer has no off-peak periods.

The standard values are defined in the section on the job-print-off-peak attribute.

Note: this document does not define how an administrator associates the off-peak names with actual time periods.

7.4.18 events-supported (#type2EnumState)

This attribute indicates the values of the job and printer notification-events attribute supported by this Printer and the states of readiness for each value.

If this attribute is unspecified, then the Printer does not support notification.

The standard values are defined in the section on the notification-events attribute.

7.4.18 locales-supported (1#type3LocaleState)

This attribute indicates the values of the job-locale attribute supported by this Printer and the states of readiness for each value.

The standard values are defined in the section on the job-locale attribute.

7.4.15 job-sheets-supported (#type3EnumState)

This attribute identifies the job-sheet -- values supported by this printer, and the state of readiness for each job-sheet.

To allow no job sheets, the system administrator shall include the value ---none as a value for this attribute. The client specifies that there are no job sheets by using the value ---none as the value of the job-sheets attribute.

If the job-sheets attribute is not specified or contains a value which the Printer does not support, --then the server shall select from among the values of this attribute.  The server shall not select the value ---none unless it is the only value specified for the job-sheets-supported attribute.

NOTE – It is preferable for the server to produce some job job-sheet, even if not the desired one, rather than produce none at all or reject the job.

-

-------

--------

-

7.4.17 maximum-copies (positiveInteger)

This attribute indicates the maximum number of copies of a document that can be rendered by this printer in a single print-job.

-----

If the attribute is unspecified or has a value of 0, there is no limit on the maximum number of copies for this Printer.

7.4.17 maximum-job-octets (positiveInteger)

This attribute indicates that the Printer shall accept a job only if its size in octets is less that the value specified by this attribute.

If the attribute is unspecified or has a value of 0, there is no limit on the size of a job in octets.

7.4.17 maximum-impressions (positiveInteger)

This attribute indicates that the Printer shall accept a job only if its size in impression is less that the value specified by this attribute.

If the attribute is unspecified or has a value of 0, there is no limit on the size of a job in impressions.

7.4.17 maximum-media-sheets (positiveInteger)

This attribute indicates that the Printer shall accept a job only if its size in media-sheets is less that the value specified by this attribute.

If the attribute is unspecified or has a value of 0, there is no limit on the size of a job in media-sheets.

7.4.17 maximum-job-retention-period (deltaTime)

This attribute indicates that when the Printer accepts a job, the retention period must not exceed the value of this attribute. Otherwise, the Printer sets the job’s retention-period to the value of this attribute.

If this attribute is unspecified, then the Printer places no limit on the retention time.

7.4.17 maximum-end-user-priority (type1Enum)

This attribute indicates that when the Printer accepts a job, the job-priority must not exceed the value of this attribute. Otherwise, the Printer sets the job’s job-priority to the value of this attribute.

If this attribute is unspecified, then the Printer places no limit on the job-priority time.

The standard values are defined in the section on the job-priority attribute.

--

--

-

-

-

7.5 Job Templates

The attributes for a Job Template  can be any of the Job object attributes defined in the sections:

Job Sheet  Attributes

Notification  Attributes

Job Scheduling Attributes

(except job-print-after)

Job Production Attributes

(except page-select)

Attributes for Conversion of Text Files

7.7 Conformance

A conforming implementation shall implement all operations, objects and attributes defined in this document.

IPP is explicitly designed to be extensible.  This means that in addition to the attributes defined in this specification, specific implementation instances may support not only the basic protocol as defined in this specification, but might add vendor specific extentions.

Also, for the core set of attributes listed in this specification, it is not required that a conforming server support all (standard) values of all supported attributes.  For example, it is not required that a printer implement all finishing methods indicated by the standard values.

The explicit requirement of the term “supported”, with respect to one of the attributes that deal with printer functions or resources, is that the server shall recognize the attribute and those values that are supported, and shall be able to respond to a query about which values that printer does, in fact, support.

8.  Security Considerations

This protocol does not identify any new security mechanisms. The authentication mechanisms (as well as extentions) built into the HTTP  infrastructure are recommended.

9.  References

[1]	Smith, R., Wright, F., Hastings, T., Zilles, S., and Gyllenskog, J., "Printer MIB", RFC 1759, March 1995.

[2]	Srinivasan, R., “RPC: Remote Procedure Call Protocol Specification Version 2", RFC 1831, August 1995.

[3]	Srinivasan, R., “XDR: External Data Representation Standard", RFC 1832, August 1995.

[4]	Postel, J., “Instructions to RFC Authors”, RFC 1543, October 1993.

[5]	ISO/IEC 10175 Document Printing Application (DPA)

[6]	Herriot, R. (editor),  X/Open A Printing System Interoperability Specification (PSIS), August 1995.

[7]	Kirk, M. (editor), POSIX System Administration -- Part 4: Printing Interfaces, POSIX 1387.4 D8, 1994.

[8]	ISO 8824 “Abstract Syntax Notation One ASN.1"

[9]	ISO 8825 “Basic Encoding Rules BER”

[10]	McLaughlin, L. III, (editor), “Line Printer Daemon Protocol” RFC 1179, August 1990.

10.  Author's Address

Scott A. Isaacson

Novell, Inc.

122 E 1700 S

Provo, UT   84606

Phone: 801-861-7366

Fax:   801-861-4025

EMail: scott_isaacson@novell.com

F. Devon Taylor

Novell, Inc.

122 E 1700 S

Provo, UT   84606

Phone: 801-861-7179

Fax:   801-861-4025

EMail: devon_taylor@novell.com

Mike MacKay

Novell, Inc.

2180 Fortune Dr.

San Jose, CA   95131

Phone: 408-577-6368

Fax:   408-577-5151

EMail: mmackay@novell.com

Peter Zehler

Xerox Corporation

800 Phillips Rd. M/S 111-01X

Webster, NY 14580

Phone: 716-265-8755

Fax:   716-265-8792

EMail: peter_zehler@wb.xerox.com

Carl-Uno Manros

Xerox Corporation

701 S. Aviation Blvd.

El Segundo, CA   90245

Phone: 310-333-8273

Fax:   310-333-5514

EMail: cmanros@cp10.es.xerox.com

Tom Hastings

Xerox Corporation

701 S. Aviation Blvd.

El Segundo, CA   90245

Phone: 310-333-6413

Fax:   310-333-5514

EMail: hastings@cp10.es.xerox.com

Robert Herriot

Sun Microsystems Inc.

2550 Garcia Ave., MPK-17

Mountain View, CA 94043

Phone: 415-786-8995

Fax:	 415-786-7077

Email: robert.herriot@eng.sun.com