Internet Printing Protocol Working Group                    Tom Hastings

INTERNET DRAFT                                                     Xerox

Expires 8 February 2001                                     Ira McDonald

High North

8 August 2000

Internet Printing Protocol (IPP):

Notifications over SNMP via Job Monitoring MIB Traps

<draft-ietf-ipp-not-over-snmp-04.txt>

Copyright (C) The Internet Society (2000). All Rights Reserved.

Status of this Memo

This document is an Internet-Draft and is in full conformance with

all provisions of Section 10 of RFC2026.  Internet-Drafts are working

documents of the Internet Engineering Task Force (IETF), its areas,

and its working groups.  Note that other groups may also distribute

working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

To view the list of Internet-Draft Shadow Directories, see

http://www.ietf.org/shadow.html.

Abstract

This document is a submission to the Internet Printing Protocol

Working Group of the Internet Engineering Task Force (IETF).  This

document is intended for eventual publication as an IETF

Informational RFC.  Comments should be submitted to the ipp@pwg.org

mailing list.

This document specifies an OPTIONAL mapping of IPP notifications over

SNMP via new service and job trap extensions to the original Job

Monitoring MIB [RFC-2707].  This mapping may be used to deliver

printer notifications for any printer (not just IPP-capable ones) and

also to deliver job notifications for any job (not just ones

submitted via IPP).

This document specifies (4) new object groups and (4) new SNMP traps

for addition to the Job Monitoring MIB [RFC 2707].  A working copy of

these extensions inserted in the original Job Monitoring MIB ASN.1 is

at:

ftp://ftp.pwg.org/pub/pwg/ipp/new_NOT/jmp-trap-000808.mib

Hastings, McDonald           Expires 8 February 2001            [Page 1]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

Table of Contents

1.  Introduction ...............................................       4

2.  Terminology ................................................       4

2.1.  Terminology for Conformance ............................       4

2.2.  SNMP Network Management Framework ......................       4

3.  Model and Operation ........................................       5

3.1.  SNMP Mapping for IPP Notifications .....................       5

3.1.1.  SNMP Mapping for IPP Printer Events ................       5

3.1.2.  SNMP Mapping for IPP Job Events ....................       6

3.2.  Subscription for IPP Notifications .....................       7

3.2.1.  Subscription via IPP ...............................       7

3.2.2.  Subscription via SNMP ..............................       7

3.3.  Relationship to other MIBs .............................       8

3.3.1.  MIB-II (RFC 1213) ..................................       8

3.3.2.  Host Resources MIB (RFC 2790) ......................       8

3.3.3.  Printer MIB (RFC 1759) .............................       8

4.  General Information ........................................       9

4.1.  Table 1 - Information about the Delivery Method ........       9

5.  Subscription Template Attributes ...........................      12

5.1.  Table 2 - Subscription and Printer Attributes ..........      12

5.2.  Additional Subscription Template Attributes ............      12

5.2.1.  notify-snmp-version (type2 keyword) ................      12

5.2.1.1.  notify-snmp-version-default ....................      13

5.2.1.2.  notify-snmp-version-supported ..................      13

5.2.2.  notify-snmp-auth-data (octetString(MAX)) ...........      13

5.2.2.1.  notify-snmp-auth-data-default ..................      14

5.2.2.2.  notify-snmp-auth-data-supported ................      14

5.2.3.  notify-snmp-operation (type2 keyword) ..............      14

5.2.3.1.  notify-snmp-operation-default ..................      15

5.2.3.2.  notify-snmp-operation-supported ................      15

5.2.4.  notify-snmp-mtu-size (integer(0:MAX)) ..............      15

5.2.4.1.  notify-snmp-mtu-size-default ...................      15

5.2.4.2.  notify-snmp-mtu-size-supported .................      16

5.3.  Additional Info about Subscription Template Attributes .      16

5.3.1.  notify-recipient-uri (uri) .........................      16

6.  Event Notification Content .................................      17

6.1.  SNMP Message Envelope ..................................      17

6.1.1.  SNMPv1 Message Envelope ............................      17

6.1.1.1.  version (INTEGER) ..............................      17

6.1.1.2.  community (OCTET STRING) .......................      17

6.1.2.  SNMPv2 Message Envelope ............................      17

6.1.2.1.  version (INTEGER) ..............................      17

6.1.2.2.  community (OCTET STRING) .......................      17

6.1.3.  SNMPv3 Message Envelope ............................      17

6.1.3.1.  msgVersion (INTEGER) ...........................      18

6.1.3.2.  msgGlobalData (HeaderData) .....................      18

6.1.3.3.  msgSecurityParameters (OCTET STRING) ...........      18

6.1.3.4.  msgData (ScopedPduData) ........................      18

6.2.  SNMP Message Header ....................................      18

6.2.1.  SNMPv1 Trap Message Header .........................      18

Hastings, McDonald           Expires 8 February 2001            [Page 2]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

6.2.1.1.  enterprise (OBJECT IDENTIFIER) .................      19

6.2.1.2.  agent-addr (NetworkAddress) ....................      19

6.2.1.3.  generic-trap (INTEGER) .........................      19

6.2.1.4.  specific-trap (INTEGER) ........................      19

6.2.1.5.  time-stamp (TimeTicks) .........................      19

6.2.2.  SNMPv2 and SNMPv3 Trap/Inform Message Header .......      19

6.2.2.1.  request-id (Integer32) .........................      19

6.2.2.2.  error-status (INTEGER) .........................      19

6.2.2.3.  error-index (INTEGER) ..........................      20

6.3.  SNMP Message Body ......................................      20

6.3.1.  SNMPv1 Trap Message Body ...........................      20

6.3.2.  SNMPv2 and SNMPv3 Trap/Inform Message Body .........      20

6.3.2.1.  sysUpTime (TimeTicks) ..........................      20

6.3.2.2.  sysTrapOID (OBJECT IDENTIFIER) .................      20

6.4.  SNMP Trap/Inform Examples ..............................      20

7.  SNMP ASN.1 for IPP Notifications ...........................      21

7.1.  Notification Definitions ...............................      21

7.1.1.  Service Event Notify Group - Trap ..................      21

7.1.2.  Job Event Notify Group - Trap ......................      22

7.1.3.  Job Completed Notify Group - Trap ..................      23

7.1.4.  Job Progress Notify Group - Trap ...................      24

7.2.  Object Definitions .....................................      25

7.2.1.  Service Group - Objects ............................      25

7.2.2.  Service Event Group - Objects ......................      28

7.2.3.  Job Event Group - Objects ..........................      32

7.2.4.  Job Progress Group - Objects .......................      35

8.  Conformance ................................................      37

9.  IANA Considerations ........................................      37

10.  Internationalization Considerations .......................      37

11.  Security Considerations ...................................      37

12.  References ................................................      38

13.  Change Log ................................................      40

14.  Intellectual Property Notice ..............................      43

15.  Authors' Addresses ........................................      43

16.  Full Copyright Statement ..................................      44

Hastings, McDonald           Expires 8 February 2001            [Page 3]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

1.  Introduction

This document is a submission to the Internet Printing Protocol

Working Group of the Internet Engineering Task Force (IETF).

Comments should be submitted to the ipp@pwg.org mailing list.

This document specifies an OPTIONAL mapping of IPP notifications over

SNMP via new service and job trap extensions to the original Job

Monitoring MIB [RFC-2707].  This mapping may be used to deliver

printer notifications for any printer (not just IPP-capable ones) and

also to deliver job notifications for any job (not just ones

submitted via IPP).

This document specifies (4) new object groups and (4) new SNMP traps

for addition to the Job Monitoring MIB [RFC 2707].  A working copy of

these extensions inserted in the original Job Monitoring MIB ASN.1 is

at:

ftp://ftp.pwg.org/pub/pwg/ipp/new_NOT/jmp-trap-000808.mib

The IPP protocol [IPP-PRO] supports passive monitoring of IPP Printer

and Job objects, via client polling of IPP object attributes using

the 'Get-Printer-Attributes' and 'Get-Job-Attributes' operations.

This document specifies support for dynamic monitoring of IPP Printer

and Job objects via SNMP traps generated by the service provider

(server or device).

2.  Terminology

2.1.  Terminology for Conformance

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted per [RFC-2119].

2.2.  SNMP Network Management Framework

See:  Section 1.1 'SNMPv1', section 1.2 'SNMPv2', and section 1.3

'SNMPv3' of [RFC-2576].

Hastings, McDonald           Expires 8 February 2001            [Page 4]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

3.  Model and Operation

3.1.  SNMP Mapping for IPP Notifications

3.1.1.  SNMP Mapping for IPP Printer Events

IPP Printer event attribute         Job Monitoring MIB object mapping

---------------------------         ---------------------------------

version-number                      [no mapping - not useful]

status-code                         [implicit in each generated trap]

notify-sequence-number              [request-id in SNMP trap header]

notify-charset                      [no mapping - strings are UTF-8]

notify-natural-language             [no mapping - no text bindings]

subscription-id                     [no mapping]

notify-subscribed-event             jmServiceEventNotifyTriggerEvent

jmServiceEventNotifyGroupEvent

notify-printer-uri                  jmServiceURI

printer-up-time                     [sysUpTime in SNMP trap bindings]

printer-current-time                [hrSystemDate in Host Res MIB]

notify-user-data                    [no mapping]

notify-text                         [no mapping]

printer-state                       jmServiceState

printer-state-reasons               jmServiceStateReasons

printer-is-accepting-jobs           [jmServiceStateReasons keyword]

Hastings, McDonald           Expires 8 February 2001            [Page 5]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

3.1.2.  SNMP Mapping for IPP Job Events

IPP Job event attribute             Job Monitoring MIB object mapping

-----------------------             ---------------------------------

version-number                      [no mapping - not useful]

status-code                         [implicit in each generated trap]

notify-sequence-number              [request-id in SNMP trap header]

notify-charset                      [no mapping - strings are UTF-8]

notify-natural-language             [no mapping - no text bindings]

subscription-id                     [no mapping]

notify-subscribed-event             jmJobEventNotifyTriggerEvent

jmJobEventNotifyGroupEvent

notify-printer-uri                  jmServiceURI

job-id                              [via jmJobState OID instance]

printer-up-time                     [sysUpTime in SNMP trap bindings]

printer-current-time                [hrSystemDate in Host Res MIB]

notify-user-data                    [no mapping]

notify-text                         [no mapping]

job-state                           jmJobState

job-state-reasons                   jmJobEventJobStateReasons

[jmJobStateReasons1 plus zero or]

[more jobStateReasonsN in a strin

[job-completed and job-purged - extra bindings]

job-k-octets-processed              jmJobKOctetsProcessed

job-impressions-completed           jmJobImpressionsCompleted

[job-progress - extra bindings]

job-k-octets                        jmJobKOctetsPerCopyRequested

job-impressions                     jmJobImpressionsPerCopyRequested

job-copies                          jmProgressJobCopiesRequested

job-collation-type                  jmProgressJobCollationType

job-media-sheets-completed          jmProgressMediaSheetsCompleted

sheet-completed-copy-number         jmProgressSheetCompletedCopyNum

sheet-completed-document-number     jmProgressSheetCompletedDocNum

Hastings, McDonald           Expires 8 February 2001            [Page 6]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

3.2.  Subscription for IPP Notifications

IPP Clients MAY subscribe for IPP Notifications delivered via SNMP by

either of the following two standard methods:

3.2.1.  Subscription via IPP

IPP Job creation (Create-Job, Print-Job, Print-URI) and

Create-Subscription operations MAY be used to create per-Printer or

per-Job IPP Subscription objects and MAY specify:

"notify-recipient-uri" = 'snmpnotify://hostname[.port]'

"notify-events" = <events of interest, e.g., 'job-completed'>

"notify-snmp-version" = <SNMP version and security model>

"notify-snmp-auth-data" = <authentication data, e.g., community>

"notify-snmp-operation" = <operation, e.g., SNMP Trap>

"notify-snmp-mtu-size" = <path max transmission unit, e.g., '4084'>

3.2.2.  Subscription via SNMP

The SNMP Notification MIB and SNMP Target MIB (both in [RFC-2573])

MAY be used to create SNMP subscriptions.

To create a subscription via SNMP, the IPP Client MAY use an SNMP

Set-Request operation to create an appropriate row in the SNMP

Notification MIB with the following variable-bindings:

1)  'snmpNotifyTag'

- tag of this notification - see 'snmpTargetAddrTagList' below

2)  'snmpNotifyType'

- deliver via SNMP Trap (unacknowledged) or Inform (acknowledged)

3)  'snmpNotifyStorageType'

- persistence (volatile, nonVolatile, permanent, readOnly)

Note:  The SNMP Notification MIB uses persistence instead of the

lease duration supported in IPP Subscription objects.

To complete a subscription via SNMP, the IPP Client MAY use an SNMP

Set-Request operation to create an appropriate row in the SNMP Target

MIB with the following variable-bindings:

Hastings, McDonald           Expires 8 February 2001            [Page 7]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

1)  'snmpTargetAddrTDomain' and 'snmpTargetAddrTAddress'

- notification target (client) transport protocol and address

(corresponds to IPP 'notify-recipient-uri')

2)  'snmpTargetAddressTimeout' and 'snmpTargetAddrRetryCount'

- retry timeout and limit (for acknowledged SNMP Inform only)

3)  'snmpTargetAddrTagList'

- tags of notifications to be sent to this target (client)

4)  'snmpTargetAddrParamsEntry'

- notification security and SNMP protocol version

5)  'snmpTargetAddrStorageType'

- persistence (volatile, nonVolatile, permanent, readOnly)

Note:  The SNMP Target MIB uses persistence instead of the lease

duration supported in IPP Subscription objects.

3.3.  Relationship to other MIBs

3.3.1.  MIB-II (RFC 1213)

All SNMPv1 Trap messages include 'time-stamp' which is the value of

the 'sysUpTime' object from MIB-II [RFC-1213].

All SNMPv2 trap bindings include the 'sysUpTime' object from MIB-II

[RFC-1213].

3.3.2.  Host Resources MIB (RFC 2790)

The 'jmServiceTable' defined in this document MAY have a sparse

mapping to the 'hrDeviceTable' (same indices) in Host Resources MIB

[RFC-2790].

3.3.3.  Printer MIB (RFC 1759)

The 'jmServiceTable' defined in this document MAY have a sparse

mapping to the 'hrDeviceTable' (same indices) in Host Resources MIB

[RFC-2790], thus offering access to the Printer MIB via a

'hrDeviceIndex' value.

Hastings, McDonald           Expires 8 February 2001            [Page 8]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

4.  General Information

4.1.  Table 1 - Information about the Delivery Method

Document Method Conformance             Delivery Method Realization

Requirement

---------------------------             ---------------------------

1. What is the URL scheme name          snmpnotify

for the Delivery Method?

2. Is the Delivery Method               OPTIONAL

REQUIRED, RECOMMENDED or OPTIONAL

for an IPP Printer to support?

3. What transport and delivery          SNMP application layer

protocols does the Printer use        over UDP transport layer

to deliver the Event                  over IP network layer

Notification Content, i.e.,           over any datalink layer

what is the entire network            (or SNMP over any other

stack?                                transport stack)

4. Can several Event                    No, impossible in SNMP

Notifications be combined into        (in an interoperable fashion)

a Compound Event Notification?

5. Is the Delivery Method               By the Printer (push)

initiated by the Notification

Recipient (pull), or by the

Printer (push)?

6. Is the Event Notification            Machine Consumable

content Machine Consumable or

Human Consumable?

7. What section in this document        Section 7 'SNMP ASN.1 for

answers the following                 for IPP Notifications'.

question? For a Machine

Consumable Event Notification,

what is the representation and

encoding of values defined in

section 0 of [ipp-ntfy] and

the conformance requirements

thereof? For a Human

Consumable Event Notification,

what is the representation and

Hastings, McDonald           Expires 8 February 2001            [Page 9]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

encoding of pieces of

information defined in section

9.2 of [ipp-ntfy] and the

conformance requirements

thereof?

8. What are the latency and             The latency is minimal in

reliability of the transport          any network topology (all

and delivery protocol?                deployed transport mappings

of SNMP are connectionless).

The reliability is medium for

SNMP Trap (unacknowledged)

and high for SNMP Inform

(acknowledged and optionally

retried by the SNMP agent).

9. What are the security aspects        SNMPv1 and SNMPv2 support

of the transport and delivery         community-based security with

protocol, e.g., how it is             very weak authentication.

handled in firewalls?                 SNMPv3 (RFC 257x series)

supports user-based security

with strong authentication

and MAY traverse firewalls

(but usually does not).

10. What are the content length         SNMP messages MUST be encoded

restrictions?                         within the 'path MTU', i.e.,

the packet is NOT fragmented

by the underlying network.

In some IP networks, this

results in a max payload of

about 480 octets.

See section 5.2.4

'notify-snmp-mtu-size'.

11. What are the additional             The 'job-completed' SNMP

values or pieces of                   notification delivers

information that a Printer            'job-k-octets-processed' as

sends in an Event Notification        as well as the base required

content and the conformance           'job-impressions-completed'

requirements thereof?                 - and both are REQUIRED

(for interoperability)

12. What are the additional             See section 5.1

Subscription Template and/or          'Table 2 - Subscription

Subscription Description              and Printer Attributes'

attributes and the conformance        - all of the additional

requirements thereof?                 subscription template

attributes are REQUIRED

(for interoperability)

Hastings, McDonald           Expires 8 February 2001           [Page 10]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

13. What are the additional             See section 5.1

Printer Description attributes        'Table 2 - Subscription

and the conformance                   and Printer Attributes'

requirements thereof?                 - all of the additional

printer description

attributes are REQUIRED

(for interoperability).

Hastings, McDonald           Expires 8 February 2001           [Page 11]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

5.  Subscription Template Attributes

5.1.  Table 2 - Subscription and Printer Attributes

All systems that support this specification MUST implement the

following REQUIRED attributes defined in this specification (on the

Subscription object or the Printer object):

Subscription Attribute                  Subscription Attribute Type

Printer Attribute                       Printer Attribute Type

-----------------------------------     --------------------------

notify-snmp-version                     (type2 keyword)

notify-snmp-version-default             (type2 keyword)

notify-snmp-version-supported           (1setOf type2 keyword)

notify-snmp-auth-data                   (octetString(MAX))

notify-snmp-auth-data-default           (octetString(MAX))

notify-snmp-auth-data-supported         (boolean)

notify-snmp-operation                   (type2 keyword)

notify-snmp-operation-default           (type2 keyword)

notify-snmp-operation-supported         (1setOf type2 keyword)

notify-snmp-mtu-size                    (integer(0:MAX))

notify-snmp-mtu-size-default            (integer(0:MAX))

notify-snmp-mtu-size-supported          (rangeOfInteger(0:MAX))

Note:  The Printer attribute 'notify-snmp-auth-data-supported' has

the irregular type 'boolean'.

5.2.  Additional Subscription Template Attributes

5.2.1.  notify-snmp-version (type2 keyword)

REQUIRED - for IPP Printers to implement.

This attribute specifies the SNMP protocol version and security model

(e.g., community-based security in SNMPv1c IETF Standard [RFC-1157])

to use for delivery of this IPP Notification.

Note:  SNMP protocol versions and SNMP security models are NOT

mutually independent - therefore they are combined in this single

attribute.

Hastings, McDonald           Expires 8 February 2001           [Page 12]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

The following standard keyword values are defined:

'snmpv1-community':  The SNMPv1 protocol with community-based

security model defined in IETF Standard [RFC-1157].

'snmpv1-party':  The SNMPv1 protocol with party-based security

model defined in IETF Historic [RFC-1352].

'snmpv2-party':  The SNMPv2 protocol with party-based security

model defined in IETF Historic [RFC-1446].

'snmpv2-community':  The SNMPv2 protocol with community-based

security model defined in IETF Experimental [RFC-1902].

'snmpv2-user':  The SNMPv2 protocol with user-based security model

defined in IETF Experimental [RFC-1910].

'snmpv3-user':  The SNMPv3 protocol with user-based security model

defined in IETF Draft Standard [RFC-2574].

5.2.1.1.  notify-snmp-version-default

REQUIRED - for IPP Printers to implement.

notify-snmp-version-default (type2 keyword)

IPP Printer object attribute for IPP Notifications over SNMP.

5.2.1.2.  notify-snmp-version-supported

REQUIRED - for IPP Printers to implement.

notify-snmp-version-supported (1setOf type2 keyword)

IPP Printer object attribute for IPP Notifications over SNMP.

5.2.2.  notify-snmp-auth-data (octetString(MAX))

REQUIRED - for IPP Printers to implement.

This attribute specifies the SNMP protocol authentication data (e.g.,

a community name in SNMPv1c IETF Standard [RFC-1157]) to use for

delivery of this IPP Notification.  This data MAY be opaque (not

human-readable).

Human-readable authentication data in this attribute (e.g., an

SNMPv1c or SNMPv2c community name) MUST be encoded in the UTF-8

[RFC-2279] transform of [ISO-10646] for interoperability.

Hastings, McDonald           Expires 8 February 2001           [Page 13]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

5.2.2.1.  notify-snmp-auth-data-default

REQUIRED - for IPP Printers to implement.

notify-snmp-auth-data-default (octetString(MAX))

IPP Printer object attribute for IPP Notifications over SNMP.

5.2.2.2.  notify-snmp-auth-data-supported

REQUIRED - for IPP Printers to implement.

notify-snmp-auth-data-supported (boolean)

IPP Printer object attribute for IPP Notifications over SNMP.

Note:  The Printer attribute 'notify-snmp-auth-data-supported' has

the irregular type 'boolean'.

5.2.3.  notify-snmp-operation (type2 keyword)

REQUIRED - for IPP Printers to implement.

This attribute specifies the SNMP operation (e.g, SNMP Trap in

SNMPv1c IETF Standard [RFC-1157]) to use for delivery of this IPP

Notification.

The following standard keyword values are defined:

'trap':  An SNMP Trap PDU encoded in the version specified by

"notify-snmp-version" above.

Note:  SNMP Trap is an unconfirmed operation (i.e., the

notification recipient MUST NOT reply with an acknowledgement) and

MUST NOT be retried by the notification generator.

'inform':  An SNMP Inform PDU encoded in the version specified by

"notify-snmp-version" above.

Note:  SNMP Inform is a confirmed operation (i.e., the notification

recipient MUST reply with an acknowledgement Inform-Response) and

MAY be retried by the notification generator.

'report':  An SNMP Report PDU encoded in the version specified by

"notify-snmp-version" above

Warning:  The use of SNMP Report MAY not be interoperable.

Hastings, McDonald           Expires 8 February 2001           [Page 14]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

5.2.3.1.  notify-snmp-operation-default

REQUIRED - for IPP Printers to implement.

notify-snmp-operation-default (type2 keyword)

IPP Printer object attribute for IPP Notifications over SNMP.

5.2.3.2.  notify-snmp-operation-supported

REQUIRED - for IPP Printers to implement.

notify-snmp-operation-supported (1setOf type2 keyword)

IPP Printer object attribute for IPP Notifications over SNMP.

5.2.4.  notify-snmp-mtu-size (integer(0:MAX))

REQUIRED - for IPP Printers to implement.

This attribute specifies the SNMP path maximum transmission unit

(MTU) size in octets to use for delivery of this IPP Notification.

The SNMP path MTU is defined as the size of the complete SNMP message

envelope, header, and body - but NOT of the underlying transport,

network, and datalink headers - such that NO fragmentation is

required for the SNMP message at ANY lower layer of the protocol

stack OR on any segment of the bridged or routed delivery path

(optimally).

This attribute enhances the integrity and probability of SNMP

delivery.

This attribute may signal the need for algorithmic truncation of some

string variable bindings in the SNMP notification which MAY not fit

in very small path MTU sizes.

See:  'ifMtu' in the Interfaces group of IETF MIB-II [RFC-1213].

5.2.4.1.  notify-snmp-mtu-size-default

REQUIRED - for IPP Printers to implement.

notify-snmp-mtu-size-default (integer(0:MAX))

IPP Printer object attribute for IPP Notifications over SNMP.

Hastings, McDonald           Expires 8 February 2001           [Page 15]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

5.2.4.2.  notify-snmp-mtu-size-supported

REQUIRED - for IPP Printers to implement.

notify-snmp-mtu-size-supported (rangeOfInteger(0:MAX))

IPP Printer object attribute for IPP Notifications over SNMP.

5.3.  Additional Info about Subscription Template Attributes

5.3.1.  notify-recipient-uri (uri)

This section describes the syntax of the value of this attribute for

the 'snmpnotify' Delivery Method for IPP Notifications.  The syntax

for values of this attribute for other Delivery Methods for IPP

Notifications is defined in each method's specification.

In order to support the 'snmpnotify' Delivery Method, the Printer

MUST support the following syntax for the 'snmpnotify' Delivery

Method when the Printer uses SNMP to deliver IPP Notifications.  The

lines below use RFC 2396 syntax rules, terms, and ABNF productions:

"snmpnotify://" hostport

hostport      = host [ ":" port ]

host          = hostname | IPv4address

hostname      = *( domainlabel "." ) toplabel [ "." ]

domainlabel   = alphanum | alphanum *( alphanum | "-" ) alphanum

toplabel      = alpha | alpha *( alphanum | "-" ) alphanum

IPv4address   = 1*digit "." 1*digit "." 1*digit "." 1*digit

port          = *digit

alphanum      = alpha | digit

alpha         = lowalpha | upalpha

lowalpha = "a" | "b" | "c" | "d" | "e" | "f" | "g" | "h" | "i" |

"j" | "k" | "l" | "m" | "n" | "o" | "p" | "q" | "r" |

"s" | "t" | "u" | "v" | "w" | "x" | "y" | "z"

upalpha  = "A" | "B" | "C" | "D" | "E" | "F" | "G" | "H" | "I" |

"J" | "K" | "L" | "M" | "N" | "O" | "P" | "Q" | "R" |

"S" | "T" | "U" | "V" | "W" | "X" | "Y" | "Z"

digit    = "0" | "1" | "2" | "3" | "4" | "5" | "6" | "7" |

"8" | "9"

Hastings, McDonald           Expires 8 February 2001           [Page 16]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

6.  Event Notification Content

6.1.  SNMP Message Envelope

6.1.1.  SNMPv1 Message Envelope

The SNMPv1 [RFC-1157] message envelope for SNMPv1c (community-based

security) contains the following:

6.1.1.1.  version (INTEGER)

The SNMP protocol version (not the IPP version) for this message.

6.1.1.2.  community (OCTET STRING)

The SNMP community name (read, write, or notify) for this message.

Used for weak authentication.

6.1.2.  SNMPv2 Message Envelope

The SNMPv2 [RFC-1905] message envelope for SNMPv2c (community-based

security) [RFC-1901] contains the following:

6.1.2.1.  version (INTEGER)

The SNMP protocol version (not the IPP version) for this message.

6.1.2.2.  community (OCTET STRING)

The SNMP community name (read, write, or notify) for this message.

Used for weak authentication.

6.1.3.  SNMPv3 Message Envelope

The SNMPv2 [RFC-1905] message envelope for SNMPv3u (user-based

security) [RFC-2574] contains the following:

Hastings, McDonald           Expires 8 February 2001           [Page 17]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

6.1.3.1.  msgVersion (INTEGER)

The SNMP protocol version (not the IPP version) for this message.

6.1.3.2.  msgGlobalData (HeaderData)

Administrative parameters.  The SNMP ASN.1 definition is as follows:

HeaderData ::= SEQUENCE {

msgID      INTEGER (0..2147483647),

msgMaxSize INTEGER (484..2147483647),

msgFlags   OCTET STRING (SIZE(1)),

msgSecurityModel INTEGER (1..2147483647)

}

6.1.3.3.  msgSecurityParameters (OCTET STRING)

Security model-specific parameters.

See:  'User-based Security Model (USM) for SNMPv3' [RFC-2574].

6.1.3.4.  msgData (ScopedPduData)

Scoped message body.  The SNMP ASN.1 definition is as follows:

ScopedPduData ::= CHOICE {

plaintext    ScopedPDU,

encryptedPDU OCTET STRING  -- encrypted scopedPDU value

}

ScopedPDU ::= SEQUENCE {

contextEngineID  OCTET STRING,

contextName      OCTET STRING,

data             ANY -- e.g., PDUs as defined in RFC 1905

}

6.2.  SNMP Message Header

6.2.1.  SNMPv1 Trap Message Header

Note:  The 'request-id' (SNMP sequence number) is NOT encoded in the

SNMPv1 Trap, whereas the SNMPv2 and SNMPv3 Trap/Inform requests DO

Hastings, McDonald           Expires 8 February 2001           [Page 18]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

have 'request-id' in their message header.

6.2.1.1.  enterprise (OBJECT IDENTIFIER)

The value of 'sysObjectID' (system type) in IETF MIB-II [RFC-1213]

for the system that generated this SNMPv1 Trap.

6.2.1.2.  agent-addr (NetworkAddress)

The network address of the SNMP agent that generated this SNMPv1

Trap.

6.2.1.3.  generic-trap (INTEGER)

The base type of this SNMPv1 Trap:  always 'enterpriseSpecific' in

IPP Notifications).

6.2.1.4.  specific-trap (INTEGER)

The specific type of this SNMPv1 Trap.

6.2.1.5.  time-stamp (TimeTicks)

The value of 'sysUpTime' in IETF MIB-II [RFC-1213] when this SNMPv1

Trap was generated.

6.2.2.  SNMPv2 and SNMPv3 Trap/Inform Message Header

Note:  The 'request-id' (SNMP sequence number) is NOT encoded in the

SNMPv1 Trap, whereas the SNMPv2 and SNMPv3 Trap/Inform requests DO

have 'request-id' in their message header.

6.2.2.1.  request-id (Integer32)

The receiver-specific sequence number for this SNMP message.

See:  Section 5.4.2 'notify-sequence-number' in [IPP-NOT].

6.2.2.2.  error-status (INTEGER)

The error status associated with this SNMP message.

Hastings, McDonald           Expires 8 February 2001           [Page 19]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

6.2.2.3.  error-index (INTEGER)

Unused in SNMP Trap messages.

6.3.  SNMP Message Body

6.3.1.  SNMPv1 Trap Message Body

The SNMPv1 Trap message body consists wholly of the specified

parameters

See:  Section 7.1 'Notification Definitions'.  in the TRAP-TYPE

definition.

6.3.2.  SNMPv2 and SNMPv3 Trap/Inform Message Body

The SNMPv2 and SNMPv3 Trap/Inform message body consists of the

specified parameters in the NOTIFICATION-TYPE definition PRECEDED by

'sysUpTime' from IETF MIB-II [RFC-1213] and 'sysTrapOID'.

See:  Section 7.1 'Notification Definitions'.

6.3.2.1.  sysUpTime (TimeTicks)

The value of 'sysUpTime' in IETF MIB-II [RFC-1213] when this SNMPv2

or SNMPv3 Trap/Inform was generated.

6.3.2.2.  sysTrapOID (OBJECT IDENTIFIER)

The specific type of this SNMPv2 or SNMPv3 Trap/Inform.  The value of

the assignment statement at the end of the NOTIFICATION-TYPE

definition.

6.4.  SNMP Trap/Inform Examples

[To be supplied]

Hastings, McDonald           Expires 8 February 2001           [Page 20]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

7.  SNMP ASN.1 for IPP Notifications

7.1.  Notification Definitions

7.1.1.  Service Event Notify Group - Trap

--  Service Event Notify Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send Service traps via SNMP.

--  The jmServiceEventNotifyGroup consists entirely of the

--  jmServiceEventV2Notify notification.

jmServiceEventV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a Service basic event sent by this managed system."

::= { jobmonMIBNotifications 1 }

jmServiceEventV2NotifyPrefix

OBJECT IDENTIFIER ::= { jmServiceEventV1Enterprise 0 }

jmServiceEventV2Notify NOTIFICATION-TYPE

OBJECTS {

jmServiceEventNotifyTriggerEvent,

jmServiceEventNotifyGroupEvent,

jmServiceState,

jmServiceStateReasons

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to an IPP Printer basic event.

The value of 'jmServiceIndex' for

use with 'jmServiceTable' for this Service is conveyed in the

instance qualifier (OID suffix) of 'jmServiceState'.

This trap is sent when requested by a prior subscription.

The specific event is in 'jmServiceEventNotifyTriggerEvent'.

The group event is in 'jmServiceEventNotifyGroupEvent'.

Additional variable-bindings SHOULD be appended to this trap:

- Systems with the Host Resources MIB (RFC 2790) SHOULD add

Hastings, McDonald           Expires 8 February 2001           [Page 21]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

'hrSystemDate'

(compare to IPP 'printer-current-time')

Additional variable-bindings MAY be appended to this trap

for all printer-specific events:

- Systems with the Host Resources MIB (RFC 2790) MAY add

'hrDeviceStatus'

(compare to IPP 'printer-state')

'hrPrinterStatus'

(compare to IPP 'printer-state') and

'hrPrinterDetectedErrorState'

(compare to IPP 'printer-state-reasons')

Systems MAY add other variable-bindings from any MIB.

See:    Standard events in 'jmServiceEventNotifyTriggerEvent';

Section 5.3.2 'notify-events' in [IPP-NOT];

Section 8.1 'notify-subscribed-event' in [IPP-NOT];

Section 9 'Event Notification Content' in [IPP-NOT]."

::= { jmServiceEventV2NotifyPrefix 1 }

7.1.2.  Job Event Notify Group - Trap

--  Job Event Notify Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send this Job trap via SNMP.

--  The jmJobEventNotifyGroup consists entirely of the

--  jmJobEventV2Notify notification.

jmJobEventV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a Job basic event sent by this managed system."

::= { jobmonMIBNotifications 2 }

jmJobEventV2NotifyPrefix

OBJECT IDENTIFIER ::= { jmJobEventV1Enterprise 0 }

jmJobEventV2Notify NOTIFICATION-TYPE

OBJECTS {

jmJobEventNotifyTriggerEvent,

jmJobEventNotifyGroupEvent,

jmJobState,

jmJobEventJobStateReasons

}

STATUS      current

Hastings, McDonald           Expires 8 February 2001           [Page 22]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

DESCRIPTION

"This SMIv2 trap corresponds to an IPP Job basic event.

The values of 'jmGeneralJobSetIndex' and 'jmJobIndex' for

use with 'jmJobTable' for this Job are conveyed in the

instance qualifier (OID suffix) of 'jmJobState'.

This trap is sent when requested by a prior subscription.

The specific event is in 'jmJobEventNotifyTriggerEvent'.

The group event is in 'jmJobEventNotifyGroupEvent'.

Additional variable-bindings SHOULD be appended to this trap:

- Systems with the Host Resources MIB (RFC 2790) SHOULD add

'hrSystemDate'

(compare to IPP 'printer-current-time')

Systems MAY add other variable-bindings from any MIB.

See:    Standard events in 'jmJobEventNotifyTriggerEvent';

Section 5.3.2 'notify-events' in [IPP-NOT];

Section 8.1 'notify-subscribed-event' in [IPP-NOT];

Section 9 'Event Notification Content' in [IPP-NOT]."

::= { jmJobEventV2NotifyPrefix 1 }

7.1.3.  Job Completed Notify Group - Trap

--  Job Completed Notify Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send this Job trap via SNMP.

jmJobCompletedV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a Job completed event sent by this managed system."

::= { jobmonMIBNotifications 3 }

jmJobCompletedV2NotifyPrefix

OBJECT IDENTIFIER ::= { jmJobCompletedV1Enterprise 0 }

jmJobCompletedV2Notify NOTIFICATION-TYPE

OBJECTS {

jmJobState,

jmJobEventJobStateReasons,

jmJobKOctetsProcessed,

jmJobImpressionsCompleted

}

STATUS      current

Hastings, McDonald           Expires 8 February 2001           [Page 23]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

DESCRIPTION

"This SMIv2 trap corresponds to an IPP 'job-completed' event.

The values of 'jmGeneralJobSetIndex' and 'jmJobIndex' for

use with 'jmJobTable' for this Job are conveyed in the

instance qualifier (OID suffix) of 'jmJobState'.

This trap is sent when requested by a prior subscription.

The event type is 'job-completed'.

Additional variable-bindings SHOULD be appended to this trap:

- Systems with the Host Resources MIB (RFC 2790) SHOULD add

'hrSystemDate'

(compare to IPP 'printer-current-time')

Systems MAY add other variable-bindings from any MIB.

See:    Section 5.3.2 'notify-events' in [IPP-NOT];

Section 8.1 'notify-subscribed-event' in [IPP-NOT];

Section 9 'Event Notification Content' in [IPP-NOT]."

::= { jmJobCompletedV2NotifyPrefix 1 }

7.1.4.  Job Progress Notify Group - Trap

--  Job Progress Notify Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send this Job trap via SNMP.

jmJobProgressV1Enterprise OBJECT-IDENTITY

STATUS      current

DESCRIPTION

"The value of the enterprise-specific OID in an SNMPv1 trap

for a Job progress event sent by this managed system."

::= { jobmonMIBNotifications 4 }

jmJobProgressV2NotifyPrefix

OBJECT IDENTIFIER ::= { jmJobProgressV1Enterprise 0 }

jmJobProgressV2Notify NOTIFICATION-TYPE

OBJECTS {

jmJobKOctetsPerCopyRequested,

jmJobKOctetsProcessed,

jmJobImpressionsPerCopyRequested,

jmJobImpressionsCompleted,

jmProgressJobCopiesRequested,

jmProgressJobCollationType,

jmProgressMediaSheetsCompleted,

jmProgressSheetCompletedCopyNum,

Hastings, McDonald           Expires 8 February 2001           [Page 24]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

jmProgressSheetCompletedDocNum

}

STATUS      current

DESCRIPTION

"This SMIv2 trap corresponds to an IPP 'job-progress' event.

The values of 'jmGeneralJobSetIndex' and 'jmJobIndex' for

use with 'jmJobTable' for this Job are conveyed in the

instance qualifier (OID suffix) of 'jmJobKOctetsProcessed'.

This trap is sent when requested by a prior subscription.

The event type is 'job-progress'.

Additional variable-bindings SHOULD be appended to this trap:

- Systems with the Host Resources MIB (RFC 2790) SHOULD add

'hrSystemDate'

(compare to IPP 'printer-current-time')

Systems MAY add other variable-bindings from any MIB.

See:    Section 5.3.2 'notify-events' in [IPP-NOT];

Section 8.1 'notify-subscribed-event' in [IPP-NOT];

Section 9 'Event Notification Content' in [IPP-NOT]."

::= { jmJobProgressV2NotifyPrefix 1 }

7.2.  Object Definitions

7.2.1.  Service Group - Objects

--  The Service Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which show Service state via SNMP.

--  The jmServiceGroup consists entirely of the jmServiceTable

jmService  OBJECT IDENTIFIER ::= { jobmonMIBObjects 7 }

jmServiceTable  OBJECT-TYPE

SYNTAX      SEQUENCE OF JmServiceEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The jmServiceTable consists of basic service state and status

information for each service which offers one or more job

services on this managed system.

Hastings, McDonald           Expires 8 February 2001           [Page 25]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

An entry SHALL exist in this table for each service, no matter

what the state of that service.  A service MAY support multiple

configured job sets and configured devices.

See:    'jmServiceJobSetsConfigured' and

'jmServiceDevicesConfigured' bit-arrays in this MIB."

::= { jmService 1 }

jmServiceEntry  OBJECT-TYPE

SYNTAX      JmServiceEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Basic service state and status information."

INDEX  { jmServiceIndex }

::= { jmServiceTable 1 }

JmServiceEntry ::= SEQUENCE {

jmServiceIndex                      Integer32 (1..2147483647),

jmServiceName                       JmUTF8StringTC (SIZE (0..63)),

jmServiceURI                        JmUTF8StringTC (SIZE (0..63)),

jmServiceJobServiceTypes            JmJobServiceTypesTC,

jmServiceJobSetsConfigured          OCTET STRING (SIZE (0..255)),

jmServiceDevicesConfigured          OCTET STRING (SIZE (0..255)),

jmServiceState                      JmServiceStateTC,

jmServiceStateReasons               SnmpAdminString (SIZE (0..255))

}

jmServiceIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The unique identifier for this service on this managed system.

See:    'jmServiceEventServiceIndex' object in this MIB."

::= { jmServiceEntry 1 }

jmServiceName OBJECT-TYPE

SYNTAX      JmUTF8StringTC (SIZE(0..63))    -- 127 in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The human readable name of this managed service.

See:    'deviceNameRequested' job attribute in this MIB;

'physicalDevice' job attribute in this MIB;

Section 4.4.4 'printer-name' in [IPP-MOD]."

Hastings, McDonald           Expires 8 February 2001           [Page 26]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

DEFVAL      { ''H }                 -- no service name

::= { jmServiceEntry 2 }

jmServiceURI OBJECT-TYPE

SYNTAX      JmUTF8StringTC (SIZE(0..63))    -- 1023 in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A URI for this managed service (valid for job services).

See:    Section 4.3.3 'job-printer-uri' in [IPP-MOD];

Section 4.4.1 'printer-uri-supported' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no service URI

::= { jmServiceEntry 3 }

jmServiceJobServiceTypes OBJECT-TYPE

SYNTAX      JmJobServiceTypesTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The types of job services supported by this managed service.

See:    'JmJobServiceTypesTC' textual convention in this MIB;

'jobServiceTypes' job attribute in this MIB."

DEFVAL      { 0 }                   -- no job services

::= { jmServiceEntry 4 }

jmServiceJobSetsConfigured OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"A bit-array that specifies the job sets configured for this

service, where each bit '2**n' is set if 'jmGeneralJobSetIndex'

is a configured job set.   Uses network byte order (big-endian)

rules - the high-order bit of the first octet corresponds to

'jmGeneralJobSetIndex' of '0' (reserved) - the low-order bit of

the first octet corresponds to 'jmGeneralJobSetIndex' of '7'.

Supports values of 'jmGeneralJobSetIndex' from '1' to '2039'.

Compare to the BITS pseudotype defined in IETF SMIv2 (RFC 2578)

which has the same bit ordering rules (big-endian).

See:    'queueNameRequested' job attribute in this MIB;

'jmGeneralJobSetIndex' table index in this MIB."

DEFVAL      { ''H }                 -- no job sets configured

::= { jmServiceEntry 5 }

jmServiceDevicesConfigured OBJECT-TYPE

SYNTAX      OCTET STRING (SIZE(0..255))

MAX-ACCESS  read-only

Hastings, McDonald           Expires 8 February 2001           [Page 27]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

STATUS      current

DESCRIPTION

"A bit-array that specifies the devices configured for this

service, where each bit '2**n' is set if 'hrDeviceIndex'

is a configured device.   Uses network byte order (big-endian)

rules - the high-order bit of the first octet corresponds to

'hrDeviceIndex' of '0' (reserved) - the low-order bit of

the first octet corresponds to 'hrDeviceIndex' of '7'.

Supports values of 'hrDeviceIndex' from '1' to '2039'.

Compare to the BITS pseudotype defined in IETF SMIv2 (RFC 2578)

which has the same bit ordering rules (big-endian).

See:    'physicalDevice' job attribute in this MIB;

'hrDeviceIndex' in IETF Host MIB (RFC 2790)."

DEFVAL      { ''H }                 -- no devices configured

::= { jmServiceEntry 6 }

jmServiceState OBJECT-TYPE

SYNTAX      JmServiceStateTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The current state of this managed service.

See:    'jmServiceEventServiceState' object in this MIB;

'jmJobState' object in this MIB;

Section 4.4.11 'printer-state' in [IPP-MOD]."

DEFVAL      { unknown }             -- unknown service state

::= { jmServiceEntry 7 }

jmServiceStateReasons OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255)) -- multi-valued in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The service state reasons associated with this service state

(as a comma-separated list) or the empty string (if none).

See:    'jmServiceEventServiceStateReasons' object in this MIB;

'jmJobStateReasons1' object in this MIB;

Section 4.4.12 'printer-state-reasons' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no service state reasons

::= { jmServiceEntry 8 }

7.2.2.  Service Event Group - Objects

--  The Service Event Group (CONDITIONALLY MANDATORY)

--

Hastings, McDonald           Expires 8 February 2001           [Page 28]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which show Service events via SNMP.

--  The jmServiceEventGroup consists entirely of the jmServiceEventTable

jmServiceEvent  OBJECT IDENTIFIER ::= { jobmonMIBObjects 8 }

jmServiceEventTable  OBJECT-TYPE

SYNTAX      SEQUENCE OF JmServiceEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The jmServiceEventTable contains service event entries for the

services which offer job services on this managed system."

::= { jmServiceEvent 1 }

jmServiceEventEntry  OBJECT-TYPE

SYNTAX      JmServiceEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Basic service event information."

INDEX  { jmServiceEventIndex }

::= { jmServiceEventTable 1 }

JmServiceEventEntry ::= SEQUENCE {

jmServiceEventIndex                 Integer32 (1..2147483647),

jmServiceEventNotifyTriggerEvent    SnmpAdminString (SIZE (0..63)),

jmServiceEventNotifyGroupEvent      SnmpAdminString (SIZE (0..63)),

jmServiceEventNotifyTime            TimeTicks,

jmServiceEventServiceIndex          Integer32 (1..2147483647),

jmServiceEventServiceState          JmServiceStateTC,

jmServiceEventServiceStateReasons   SnmpAdminString (SIZE (0..255))

}

jmServiceEventIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The unique identifier for this event on this managed system."

::= { jmServiceEventEntry 1 }

jmServiceEventNotifyTriggerEvent OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..63))   -- 255 in [IPP-NOT]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specific event that created this row in 'jmServiceEventTable'.

Hastings, McDonald           Expires 8 February 2001           [Page 29]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

For example, 'printer-stopped' here and 'printer-state-changed'

in 'jmServiceEventNotifyGroupEvent'.  Whereas a transition of

'jmServiceState' from 'idle' to 'processing' would report

'printer-state-changed' in both 'jmService...Event' objects.

Standard Printer event types defined in [IPP-NOT] are:

- 'printer-state-changed' (group event)

- 'printer-restarted'

- 'printer-shutdown'

- 'printer-stopped'

- 'printer-config-changed' (group event)

- 'printer-media-changed'

- 'printer-finishings-changed'

- 'printer-queue-order-changed' (group event)

Standard Service event types generalized from [IPP-NOT] are:

- 'service-state-changed' (group event)

- 'service-restarted'

- 'service-shutdown'

- 'service-stopped'

- 'service-config-changed' (group event)

- 'service-media-changed'

- 'service-finishings-changed'

- 'service-queue-order-changed' (group event)

- and (optionally) vendor extension event types

Conformance:    The natural language for keywords

in event type SHALL always be US English.

See:    'jmServiceEventNotifyGroupEvent' in this MIB;

Section 8.1 'notify-subscribed-event' in [IPP-NOT]."

DEFVAL      { ''H }                 -- no notify event

::= { jmServiceEventEntry 2 }

jmServiceEventNotifyGroupEvent OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..63))   -- 255 in [IPP-NOT]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Group event that created this row in 'jmServiceEventTable'.

For example, 'printer-state-changed' here and 'printer-stopped'

in 'jmServiceEventNotifyTriggerEvent'.  Whereas a transition of

'jmServiceState' from 'idle' to 'processing' would report

'printer-state-changed' in both 'jmService...Event' objects.

Conformance:    The natural language for keywords

in event type SHALL always be US English.

See:    Standard events in 'jmServiceEventNotifyTriggerEvent';

Section 8.1 'notify-subscribed-event' in [IPP-NOT]."

DEFVAL      { ''H }                 -- no notify event

Hastings, McDonald           Expires 8 February 2001           [Page 30]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

::= { jmServiceEventEntry 3 }

jmServiceEventNotifyTime OBJECT-TYPE

SYNTAX      TimeTicks

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The time of this service event.

Usage:  Conforming management agents, which MUST implement the

IETF MIB-II (RFC 1213), SHALL set 'jmServiceEventNotifyTime' to

'sysUpTime' when a service event row is created.

See:    'sysUpTime' in IETF MIB-II (RFC 1213);

Section 5.4.4 'notify-printer-up-time' in [IPP-NOT];

Section 4.4.29 'printer-up-time' in [IPP-MOD]."

::= { jmServiceEventEntry 4 }

jmServiceEventServiceIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unique identifier for this service on this managed system.

See:    'jmServiceIndex' object in this MIB."

::= { jmServiceEventEntry 5 }

jmServiceEventServiceState OBJECT-TYPE

SYNTAX      JmServiceStateTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The state of this managed service at the time of this event.

See:    'jmServiceState' object in this MIB;

'jmJobState' object in this MIB;

Section 4.4.11 'printer-state' in [IPP-MOD]."

DEFVAL      { unknown }             -- unknown service state

::= { jmServiceEventEntry 6 }

jmServiceEventServiceStateReasons OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..255)) -- multi-valued in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The service state reasons associated with this service state

(as a comma-separated list) or the empty string (if none).

See:    'jmServiceStateReasons' object in this MIB;

'jmJobStateReasons1' object in this MIB;

Hastings, McDonald           Expires 8 February 2001           [Page 31]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

Section 4.4.12 'printer-state-reasons' in [IPP-MOD]."

DEFVAL      { ''H }                 -- no service state reasons

::= { jmServiceEventEntry 7 }

7.2.3.  Job Event Group - Objects

--  The Job Event Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which show Job events via SNMP.

--  The jmJobEventGroup consists entirely of the jmJobEventTable

jmJobEvent  OBJECT IDENTIFIER ::= { jobmonMIBObjects 9 }

jmJobEventTable  OBJECT-TYPE

SYNTAX      SEQUENCE OF JmJobEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"The jmJobEventTable contains job event entries for the

jobs present on this managed system."

::= { jmJobEvent 1 }

jmJobEventEntry  OBJECT-TYPE

SYNTAX      JmJobEventEntry

MAX-ACCESS  not-accessible

STATUS      current

DESCRIPTION

"Basic job event information."

INDEX  { jmJobEventIndex }

::= { jmJobEventTable 1 }

JmJobEventEntry ::= SEQUENCE {

jmJobEventIndex                     Integer32 (1..2147483647),

jmJobEventNotifyTriggerEvent        SnmpAdminString (SIZE (0..63)),

jmJobEventNotifyGroupEvent          SnmpAdminString (SIZE (0..63)),

jmJobEventNotifyTime                TimeTicks,

jmJobEventJobSetIndex               Integer32 (1..32767),

jmJobEventJobIndex                  Integer32 (1..2147483647),

jmJobEventJobState                  JmJobStateTC,

jmJobEventJobStateReasons           OCTET STRING (SIZE (4..16))

}

jmJobEventIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  not-accessible

Hastings, McDonald           Expires 8 February 2001           [Page 32]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

STATUS      current

DESCRIPTION

"The unique identifier for this event on this managed system."

::= { jmJobEventEntry 1 }

jmJobEventNotifyTriggerEvent OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..63))   -- 255 in [IPP-NOT]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Specific event that created this row in 'jmJobEventTable'.

For example, 'job-stopped' here and 'job-state-changed'

in 'jmJobEventNotifyGroupEvent'.  Whereas a transition of

'jmJobState' from 'pending' to 'processing' would report

'job-state-changed' in both 'jmJob...Event' objects.

Standard Job event types defined in [IPP-NOT] are:

- 'job-state-changed' (group event)

- 'job-created'

- 'job-completed'

- 'job-stopped'

- 'job-config-changed' (group event)

- 'job-progress' (group event)

- and (optionally) vendor extension event types

Conformance:    The natural language for keywords

in event type SHALL always be US English.

See:    'jmJobEventNotifyGroupEvent' in this MIB;

Section 8.1 'notify-subscribed-event' in [IPP-NOT]."

DEFVAL      { ''H }                 -- no notify event

::= { jmJobEventEntry 2 }

jmJobEventNotifyGroupEvent OBJECT-TYPE

SYNTAX      SnmpAdminString (SIZE (0..63))   -- 255 in [IPP-NOT]

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"Group event that created this row in 'jmJobEventTable'.

For example, 'job-state-changed' here and 'job-stopped'

in 'jmJobEventNotifyTriggerEvent'.  Whereas a transition of

'jmJobState' from 'pending' to 'processing' would report

'job-state-changed' in both 'jmJob...Event' objects.

Conformance:    The natural language for keywords

in event type SHALL always be US English.

See:    'jmJobEventNotifyGroupEvent' in this MIB;

Section 8.1 'notify-subscribed-event' in [IPP-NOT]."

DEFVAL      { ''H }                 -- no notify event

::= { jmJobEventEntry 3 }

Hastings, McDonald           Expires 8 February 2001           [Page 33]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

jmJobEventNotifyTime OBJECT-TYPE

SYNTAX      TimeTicks

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The time of this job event.

Usage:  Conforming management agents, which MUST implement the

IETF MIB-II (RFC 1213), SHALL set 'jmJobEventNotifyTime' to

'sysUpTime' when a job event row is created.

See:    'sysUpTime' in IETF MIB-II (RFC 1213);

Section 5.4.4 'notify-printer-up-time' in [IPP-NOT];

Section 4.4.29 'printer-up-time' in [IPP-MOD]."

::= { jmJobEventEntry 4 }

jmJobEventJobSetIndex OBJECT-TYPE

SYNTAX      Integer32 (1..32767)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unique identifier for this job set on this managed system.

See:    'jmGeneralJobSetIndex' object in this MIB."

::= { jmJobEventEntry 5 }

jmJobEventJobIndex OBJECT-TYPE

SYNTAX      Integer32 (1..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The unique identifier for this job on this managed system,

when prefixed with 'jmJobEventJobSetIndex'.

See:    'jmJobIndex' object in this MIB."

::= { jmJobEventEntry 6 }

jmJobEventJobState OBJECT-TYPE

SYNTAX      JmJobStateTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The state of this managed job at the time of this event.

See:    'jmJobState' in this MIB;

Section 4.3.7 'job-state' in [IPP-MOD]."

DEFVAL      { unknown }             -- unknown job state

::= { jmJobEventEntry 7 }

jmJobEventJobStateReasons OBJECT-TYPE

Hastings, McDonald           Expires 8 February 2001           [Page 34]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

SYNTAX      OCTET STRING (SIZE (4..16)) -- multi-valued in IPP

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The job state reasons associated with this job state

represented as one to four concatenated 32-bit integers

in network byte order (big-endian).

Usage:  Conforming management stations SHALL always report the

value of the object 'jmJobStateReasons1' in the first four

octets of 'jmJobEventJobStateReasons' and SHOULD report values

of the attributes 'jobStateReasons2', 'jobStateReasons3', and

'jobStateReasons4' in subsequent octets.

See:    'jmJobStateReasons1' in this MIB;

Section 4.3.8 'job-state-reasons' in [IPP-MOD]."

DEFVAL      { '00000000'H }         -- no job state reasons

::= { jmJobEventEntry 8 }

7.2.4.  Job Progress Group - Objects

--  The Job Progress Group (CONDITIONALLY MANDATORY)

--

--  Implementation of this group is conditionally mandatory;

--  mandatory for systems which send Job progress traps via SNMP.

--  The jmProgressGroup consists entirely of leaf objects for traps

jmProgress  OBJECT IDENTIFIER ::= { jobmonMIBObjects 10 }

jmProgressJobCopiesRequested OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of copies of this job requested by the client.

See:    'jobCopiesRequested' attribute in this MIB."

DEFVAL      { -2 }                  -- unknown job copies

::= { jmProgress 1 }

jmProgressJobCollationType OBJECT-TYPE

SYNTAX      JmJobCollationTypeTC

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of copies of this job requested by the client.

See:    'jobCollationType' attribute in this MIB;

Hastings, McDonald           Expires 8 February 2001           [Page 35]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

'job-collation-type' in [IPP-PROG];

Section 9 'Event Notification Content' in [IPP-NOT]."

DEFVAL      { unknown }             -- unknown job collation type

::= { jmProgress 2 }

jmProgressMediaSheetsCompleted OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of media sheets completed for this job.

See:    'sheetsCompleted' attribute in this MIB;

Section 4.3.18.3 'job-media-sheets-completed'

in [IPP-MOD];

Section 9 'Event Notification Content' in [IPP-NOT]."

DEFVAL      { -2 }                  -- unknown job progress

::= { jmProgress 3 }

jmProgressSheetCompletedCopyNum OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of the job copy currently being stacked for the

current document or zero (if none) or '-2' (unknown).

See:    'sheetCompletedCopyNumber' attribute in this MIB;

'sheet-completed-copy-number' in [IPP-PROG];

Section 9 'Event Notification Content' in [IPP-NOT]."

DEFVAL      { -2 }                  -- unknown sheet complete copy

::= { jmProgress 4 }

jmProgressSheetCompletedDocNum OBJECT-TYPE

SYNTAX      Integer32 (-2..2147483647)

MAX-ACCESS  read-only

STATUS      current

DESCRIPTION

"The number of the job document currently being stacked for the

current job or zero (if none) or '-2' (unknown).

See:    'sheetCompletedDocumentNumber' attribute in this MIB;

'sheet-completed-document-number' in [IPP-PROG];

Section 9 'Event Notification Content' in [IPP-NOT]."

DEFVAL      { -2 }                  -- unknown sheet complete doc

::= { jmProgress 5 }

Hastings, McDonald           Expires 8 February 2001           [Page 36]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

8.  Conformance

See DESCRIPTION clauses of objects and notifications defined in section

7 'SNMP ASN.1 for IPP Notifications'.

9.  IANA Considerations

None.

10.  Internationalization Considerations

The 'jmServiceStateReasons', 'jmServiceEventServiceStateReasons',

'jmServiceEventNotify[Trigger|Group]Event', and

'jmJobEventNotify[Trigger|Group]Event' text string objects defined in

this document have a syntax (datatype) of 'SnmpAdminString' (UTF-8

stream-encoded ISO 10646/Unicode text) defined in the SNMPv3

Framework MIB [RFC-2571].  The natural language of these objects is

US English (they contain keywords which are protocol elements).

11.  Security Considerations

This IPP Notifications over SNMP mapping defines only 'read-only'

objects.  It is suitable for use with any version of SNMP, as no

update security is required (because no configuration updates are

supported).

No sensitive information is available via IPP Notifications over

SNMP.

Hastings, McDonald           Expires 8 February 2001           [Page 37]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

12.  References

[IPP-MOD] R. deBry, T. Hastings, R. Herriot, S. Isaacson, P. Powell.

IPP/1.1 Model and Semantics, <draft-ietf-ipp-model-v11-07.txt>

(work-in-progress), May 2000.

[IPP-NOT] S. Isaacson, J. Martin, R. deBry, T. Hastings, M. Shepherd,

R. Bergman.  IPP Event Notification Specification,

<draft-ietf-ipp-not-spec-04.txt> (work-in-progress), July 2000.

[IPP-PROG] T. Hastings, R. Bergman, H. Lewis.  Proposed Job Progress

Attributes for IPP, <draft-ietf-ipp-job-prog-00.txt>

(work-in-progress), July 2000.

[RFC-1157] J.D. Case, M. Fedor, M.L. Schoffstall, C. Davin.  Simple

Network Management Protocol, RFC 1157, May 1990.

[RFC-1213] K. McCloghrie, M. Rose.  MIB-II, RFC 1213, March 1991.

[RFC-1215] M. Rose.  Convention for Defining Traps for SNMPv1, RFC

1215, March 1991.

[RFC-1759] R. Smith, F. Wright, T. Hastings, S. Zilles,

J. Gyllenskog.  Printer MIB, RFC 1759, March 1995.

[RFC-1901] J. Case, K. McCloghrie, M. Rose, S. Waldbusser.

Introduction to Community-based SNMPv2, RFC 1901, January 1996.

[RFC-1905] J. Case, K. McCloghrie, M. Rose, S. Waldbusser.  Protocol

Operations for SNMPv2, RFC 1905, January 1996.

[RFC-2396] T. Berners-Lee, R. Fielding, L. Masinter.  Uniform

Resource Identifiers (URI):  Generic Syntax, RFC 2396, August 1998.

[RFC-2571] D. Harrington, R. Presuhn, B. Wijnen.  Architecture for

SNMP Management Frameworks, RFC 2571, April 1999.

[RFC-2573] D. Levi, P. Meyer, B. Stewart.  SNMP Applications, RFC

2573, April 1999.

[RFC-2574] U. Blumenthal, B. Wijnen.  User-based Security Model (USM)

for SNMPv3, RFC 2574, April 1999.

[RFC-2575] B. Wijnen, R. Presuhn, K. McCloghrie.  View-based Access

Control Model (VACM) for SNMPv3, RFC 2575, April 1999.

[RFC-2576] R. Frye, D. Levi, S. Routhier, B. Wijnen.  Coexistence

between Version 1, Version 2, and Version 3 of the Internet-standard

Network Management Framework, RFC 2576, March 2000.

Hastings, McDonald           Expires 8 February 2001           [Page 38]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

[RFC-2707] R. Bergman, T. Hastings, S. Isaacson, H. Lewis.  Job

Monitoring MIB v1.0, RFC 2707, November 1999

[RFC-2790] P. Grillo, S. Waldbusser.  Host Resources MIB, RFC 2790,

March 2000.

Hastings, McDonald           Expires 8 February 2001           [Page 39]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

13.  Change Log

[This section will be deleted before publication as an RFC]

Changes in reverse chronological order (most recent first).

<draft-ietf-ipp-not-over-snmp-04.txt> - 8 August 2000

1)  Deleted 'printer-full', 'printer-no-longer-full',

'printer-almost-idle', and 'printer-not-almost-idle' event types,

and added 'printer-stopped' in 'jmServiceEventNotifyTriggerEvent',

for alignment with revised [IPP-NOT];

2)  Deleted 'job-purged' event type and added 'job-stopped' in

'jmJobEventNotifyTriggerEvent', for alignment with revised

[IPP-NOT];

3)  Renamed 'jmServiceEventNotifyEvent' object to

'jmServiceEventNotifyTriggerEvent' (most specific event) and added

'jmServiceEventNotifyGroupEvent' (most general event), for higher

fidelity mapping to IPP 'notify-subscribed-event';

4)  Renamed 'jmJobEventNotifyEvent' object to

'jmJobEventNotifyTriggerEvent' (most specific event) and added

'jmJobEventNotifyGroupEvent' (most general event), for higher

fidelity mapping to IPP 'notify-subscribed-event';

5)  Renamed all NOTIFICATION-TYPEs (traps) for clarity, changing

'jmServiceBasicV2Event' to 'jmServiceEventV2Notify',

'jmJobBasicV2Event' to 'jmJobEventV2Notify',

'jmJobCompletedV2Event' to 'jmJobCompletedV2Notify', and

'jmJobProgressV2Event' to 'jmJobProgressV2Notify'.

<draft-ietf-ipp-not-over-snmp-03.txt> - 6 July 2000

1)  Added 'SnmpAdminString' to IMPORTS clause for new objects.

2)  Corrected OID in MODULE-IDENTITY to use forward reference to

definition of 'pwg' from 'enterprises' and 'mibs' from 'pwg'.

3)  Added 'JmServiceStateTC' textual convention.

4)  Added 'jmMirrorAttr' and 'jmSystem' object identifiers reserved

for future extensions.

5)  Major rewrite, per email discussion on IETF IPP WG list, to

specify four new small (traditional) SNMP traps for:

'jmServiceBasicV2Event' (generalized from IPP Printer event),

'jmJobBasicV2Event' (corresponds IPP Job event),

'jmJobCompletedV2Event' (corresponds IPP Job completed event),

'jmJobProgressV2Event' (corresponds IPP Job progress event).

6)  Major rewrite, per email discussion on IETF IPP WG list, to

specify four new SNMP object groups:  'jmServiceTable' (name, URI,

state, etc.  - from IPP Printer), 'jmServiceEventTable' (records

IPP Printer events for polling), 'jmJobEventTable' (records IPP Job

events for polling), 'jmJobProgressGroup' (leaf objects for IPP Job

progress event).

7)  Revised section 3.1 'SNMP Mapping for IPP Printer Events' and

section 3.2 'SNMP Mapping for IPP Job Events', to agree with above.

Hastings, McDonald           Expires 8 February 2001           [Page 40]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

8)  Deleted obsolete section 3.3 'Rules for Encoding Notifications',

as event bindings now always fit over all SNMP transport protocols.

<draft-ietf-ipp-not-over-snmp-02.txt> - 19 March 2000

1)  Renamed Printer Event notification group to Device Basic Event

and 'jmPrinterEventV2Event' notification to 'jmDeviceBasicV2Event',

to better align with IPP and to support non-printing jobs.

2)  Revised 'jmDeviceBasicV2Event' notification to remove

'hrDeviceStatus', 'hrPrinterStatus', 'hrPrinterDetectedErrorState'

from mandatory trap bindings because they were redundant, per

request of Ron Bergman.

3)  Renamed Job Event notification group to Job Basic Event and

'jmJobEventV2Event' notification to 'jmJobBasicV2Event', to better

align with IPP and to support variant job events.

4)  Defined new Job Completed Event notification group and defined

new Job Progress Event notification group, to better align with IPP

and to support variant job events.

5)  Renamed Event object group to Event Binding,

'jmEventPrinterState' to 'jmEventDeviceState',

'jmEventPrinterStateReasons' to 'jmEventDeviceStateReasons',

'jmEventPrinterIsAcceptingJobs' to 'jmEventDeviceIsAcceptingJobs',

to support non-printing jobs.

6)  Revised Event Binding object group, adding explicit objects

'jmEventDeviceURI', 'jmEventDeviceName', 'jmEventJobSetIndex',

'jmEventJobIndex', 'jmEventJobName', 'jmEventJobState',

'jmEventJobStateReasons', 'jmEventJobKOctets',

'jmEventJobKOctetsProcessed', 'jmEventJobImpressions',

'jmEventJobImpressionsCompleted', 'jmEventJobMediaSheets',

'jmEventJobMediaSheetsCompleted',

'jmEventJobImpressionsCompletedCC', 'jmEventJobCollationType',

'jmEventJobSheetCompletedCopyNum',

'jmEventJobSheetCompletedDocNum', per request of Ron Bergman.

7)  Revised SYNTAX of 'jmEventTriggerEvent' object from from

'JmUTF8StringTC' (string) to IPP-aligned enumeration, per request

of Ron Bergman.

8)  Removed all references to Printer MIB v2, as they were of limited

value, per request of Ron Bergman.

9)  Revised 'SNMP Mapping for IPP Printer Events' section for renamed

event binding objects, per request of Ron Bergman.

10) Revised 'Rules for Encoding Notifications' section to truncate

additional string bindings, per request of Ron Bergman.

11) Revised 'Registration via IPP' section, to change scheme name

from 'ipp-snmp:' to 'snmpnotify:', per request of Ron Bergman.

<draft-ietf-ipp-not-over-snmp-01.txt> - 1 December 1999

1)  Deleted 'JmTriggerEventTC' textual convention (see below).

2)  Revised SYNTAX of 'jmEventTriggerEvent' object from

'JmTriggerEventTC' (enumeration) to 'JmUTF8StringTC' (string), to

support use of IPP standard keywords.

3)  Added 'jmEventPrinterState', 'jmEventPrinterStateReasons', and

'jmEventPrinterIsAcceptingJobs' objects for consistency w/

Hastings, McDonald           Expires 8 February 2001           [Page 41]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

[IPP-NOT] and to reduce ambiguity about printer states inherent in

RFC 1759.

4)  Revised DESCRIPTION of 'jmPrinterEventV2Event' notification to

add SHOULD (recommendation) for 'jmEventPrinterState',

'jmEventPrinterStateReasons', and 'jmEventPrinterIsAcceptingJobs'

objects.

5)  Revised 'SNMP Mapping for IPP Printer Events' section to add

direct mapping of IPP notification attributes to

'jmEventPrinterState', 'jmEventPrinterStateReasons', and

'jmEventPrinterIsAcceptingJobs' objects.

6)  Revised 'Rules for Encoding Notifications' section to add

'jmEventPrinterState' and 'jmEventPrinterStateReasons'.

7)  Revised 'IANA Considerations' section to specify there are none -

no enumerated or keyword textual conventions are now defined in

this document.

8)  Revised 'Internationalization Considerations' section to specify

that US English keywords are used in 'jmEventTriggerEvent',

'jmEventPrinterState', and 'jmEventPrinterStateReasons' objects and

thus no explicit natural language tagging is required.

<draft-ietf-ipp-not-over-snmp-00.txt> - 10 October 1999

1)  Initial version.

Hastings, McDonald           Expires 8 February 2001           [Page 42]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

14.  Intellectual Property Notice

The IETF takes no position regarding the validity or scope of any

intellectual property or other rights that might be claimed to

pertain to the implementation or use of the technology described in

this document or the extent to which any license under such rights

might or might not be available; neither does it represent that it

has made any effort to identify any such rights.  Information on the

IETF's procedures with respect to rights in standards-track and

standards-related documentation can be found in BCP-11.  Copies of

claims of rights made available for publication and any assurances of

licenses to be made available, or the result of an attempt made to

obtain a general license or permission for the use of such

proprietary rights by implementers or users of this specification can

be obtained from the IETF Secretariat.

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights which may cover technology that may be required to practice

this standard.  Please address the information to the IETF Executive

Director.

15.  Authors' Addresses

Tom Hastings

Xerox Corporation

701 S Aviation Blvd, MS 834-03E

El Segundo, CA  90245

Phone: +1 310-333-6413

Email: hastings@cp10.es.xerox.com

Ira McDonald

High North Inc

221 Ridge Ave

Grand Marais, MI  49839

Phone: +1 906-494-2434 or +1 906-494-2697

Email: imcdonald@sharplabs.com

Email: imcdonal@sdsp.mc.xerox.com

Hastings, McDonald           Expires 8 February 2001           [Page 43]

Internet Draft      IPP Notify via Job Mon MIB Traps       8 August 2000

16.  Full Copyright Statement

Copyright (C) The Internet Society (2000).  All Rights Reserved.

This document and translations of it may be copied and furnished to

others, and derivative works that comment on or otherwise explain it

or assist in its implementation may be prepared, copied, published

and distributed, in whole or in part, without restriction of any

kind, provided that the above copyright notice and this paragraph are

included on all such copies and derivative works.  However, this

document itself may not be modified in any way, such as by removing

the copyright notice or references to the Internet Society or other

Internet organizations, except as needed for the purpose of

developing Internet standards in which case the procedures for

copyrights defined in the Internet Standards process must be

followed, or as required to translate it into languages other than

English.

The limited permissions granted above are perpetual and will not be

revoked by the Internet Society or its successors or assigns.

This document and the information contained herein is provided on an

"AS IS" basis and THE INTERNET SOCIETY AND THE INTERNET ENGINEERING

TASK FORCE DISCLAIMS ALL WARRANTIES, EXPRESS OR IMPLIED, INCLUDING

BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE INFORMATION

HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED WARRANTIES OF

MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

Hastings, McDonald           Expires 8 February 2001           [Page 44]