AlertCode Table for IBM NP12/17/24 Printers - V2.1

| prt Alert Code | Symbolic Name | Severity Level | Training Level | Group Index | Sub- Unit Index | Location Decimal | Device Status | Printer Status |  | Printer Detected Error State |  | prtAlertDescription |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Cover Open | 3 | 3 | 6 | n | N/A | 5 | 1 |  | bit 4 |  | ‘Cover Open’ |
| 3 | Cover Open | 3 | 3 | 6 | n | N/A | 5 | 1 |  | bit 4 |  | ‘Finisher Cover Open’ |
| 8 | Jammed (Staples) | 3 | 3 | 30 | 1 | N/A | 5 | 1 |  | bit 5 |  | ‘Staple Jam’ |
| 8 | Jammed (Exit Area) | 3 | 3 | 13 | 1 | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Input n) | 3 | 3 | 8 | n | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Duplex Unit) | 3 | 3 | 13 | 1 | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Fuser) | 3 | 3 | 13 | 1 | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Output n) | 3 | 3 | 9 | n | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Mailbox n) | 3 | 3 | 9 | n | N/A | 5 | 1 |  | bit 5 |  | ‘Mailbox Jam’ |
| 8 | Jammed (Input Area) | 3 | 3 | 6 | n | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Fuser Area) | 3 | 3 | 6 | n | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Face down bin) | 3 | 3 | 6 | n | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Duplex) | 3 | 3 | 6 | n | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (HiCap tray) | 3 | 3 | 8 | n | N/A | 5 | 1 |  | bit 5 |  | ‘Paper Jam’ |
| 8 | Jammed (Finisher) | 3 | 3 | 30 | 1 | N/A | 5 | 1 |  | bit 5 |  | ‘Finisher Jam’ |
| 9 | subunitMissing | 3 | 3 | 8 | n | N/A | 5 | 1 |  | bit 0 |  | ‘Tray Missing’ |
| 9 | subunitMissing | 3 | 3 | 13 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Duplex Option Error’ |
| 9 | subunitMissing | 3 | 3 | 8 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Envelope Option Error’ |
| 9 | subunitMissing (HiCap tray) | 3 | 3 | 8 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Option Error’ |
| 9 | subunitMissing | 3 | 3 | 30 | 1 | N/A | 5 | 1 |  | bit 7 |  | ‘Finisher Option Error’ |
| 13 | subunitEmpty | 3 | 3 | 8 | n | N/A | 5 | 1 |  | bit 1 |  | ‘Load Paper inTray’ |
| 13 | subunitEmpty | 3 | 3 | 30 | 1 | N/A | 5 | 1 |  | bit 7 |  | ‘Load Staples’ |
| 15 | subunitFull | 3 | 3 | 9 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Output Full’ |
| 22 | subunitOffline | 3 | 3 | 5 | -1 | N/A | 5 | 1 |  | bit 6 |  | ‘Offline’ |
| 29 | subunitRecoverableFailure | 3 | 3 | 14 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Serial COMM Error’ |
| 30 | subunitUnrecoverableFailure | 3 | 3 | 8 or 9 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Option Error’ |
| 30 | subunitUnrecoverableFailure | 3 | 3 | 9 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Mailbox Power’ |
| 30 | subunitUnrecoverableFailure | 3 | 4 | 4 | -1 | N/A | 5 | 1 |  | bit 7 |  | ‘Network Option Error’ |
| 30 | subunitUnrecoverableFailure (Laser Beam) | 3 | 5 | 10 | 1 | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 30 | subunitUnrecoverableFailure (Scanner Speed) | 3 | 5 | 10 | 1 | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 30 | subunitUnrecoverableFailure (NP24 Duplexer) | 3 | 5 | 13 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Duplexer Call for Service’ |
| 30 | subunitUnrecoverableFailure (Duplex Unit) | 3 | 5 | 13 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 30 | subunitUnrecoverableFailure | 3 | 4 | 8 | 1 | N/A | 5 | 1 |  | bit 7 |  | ‘Tray 1 Error Call for Service’ |
| 30 | subunitUnrecoverableFailure | 3 | 4 | 4 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Option Error’ |
| 30 | subunitUnrecoverableFailure | 3 | 4 | 10 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Fuser Call for Service’ |
| 30 | subunitUnrecoverableFailure | 3 | 4 | 10 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Scanner Call for Service’ |
| 30 | subunitUnrecoverableFailure | 3 | 4 | 10 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Scanner Call for Service’ |
| 30 | subunitUnrecoverableFailure | 3 | 4 | 10 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Laser Call for Service’ |
| 30 | subunitUnrecoverableFailure | 3 | 4 | 30 | 1 | N/A | 5 | 1 |  | bit 7 |  | ‘Error Finisher’ |
| 31 | subunitRecoverableStorage Error | 4 | 7 | 3 | n | N/A | 3 | 3 or 4 |  |  |  | ‘Memory Alloc Error’ |
| 32 | subunitUnrecoverable Storage Error | 3 | 3 | 3 | n | N/A | 5 | 1 |  |  |  | ‘Memory Overflow’ |
| 32 | subunitUnrecoverable Storage Error (NVRAM) | 4 | 5 | 3 | -1 | N/A | 3 | 3 or 4 |  | bit 7 |  | ‘Error Call for Service’ |
| 32 | subunitUnrecoverable Storage Error (EEPROM) | 3 | 5 | 3 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 32 | subunitUnrecoverable Storage Error | 3 | 5 | 3 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Disk Error’ |
| 33 | subunitMotorFailure (Main Motor) | 3 | 5 | 5 | -1 | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 33 | subunitMotorFailure (Fan) | 3 | 5 | 5 | -1 | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 33 | subunitMotorFailure | 3 | 5 | 10 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Scanner Call for Service’ |
| 33 | subunitMotorFailure | 3 | 5 | 30 | 1 | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 33 | subunitMotorFailure | 3 | 5 | 8 | 1 | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 503 | poweredUp | 4 | 7 | 5 | -1 | N/A | 2 | 3 or 4 |  |  |  | ‘Powered Up’ |
| 506 | printerManualReset | 4 | 7 | 5 | -1 | N/A | 2 | 3 or 4 |  |  |  | ‘Resetting Printer’ |
| 810 | inputManualInputRequest | 3 | 3 | 8 | n | N/A | 5 | 1 |  | bit 1 |  | ‘Load Paper in Manual’ |
| 812 | inputTrayElevationFailure | 3 | 5 | 8 | n | N/A | 5 | 1 |  | bit 7 |  | ‘Error Call for Service’ |
| 813 | inputCannotFeedSizeSelected | 3 | 5 | 8 | n | N/A | 5 | 1 |  | bit 1 |  | ‘Wrong Paper Size’ |
| 1001 | markerFuserUnderTemp | 3 | 5 | 10 | 1 | N/A | 5 |  | 1 |  | bit 7 | ‘Error Fuser Call for Service’ |
| 1002 | markerFuserOverTemp | 3 | 5 | 10 | 1 | N/A | 5 |  | 1 |  | bit 7 | ‘Error Call for Service’ |
| 1004 | markerFuserThermistorFailure | 3 | 5 | 10 | 1 | N/A | 5 |  | 1 |  | bit 7 | ‘Error Fuser Call for Service’ |
| 1104 | markerTonerAlmostEmpty | 4 | 3 | 11 | -2 | N/A | 3 |  | 3 or 4 |  | bit 2 | ‘Toner Low’ |
| 1115 | Toner Cartridge Missing | 3 | 3 | 11 | -2 | N/A | 5 |  | 1 |  | bit 3 | ‘Toner Cartridge Missing’ |

Severity Level

3	Critical Binary

4	Warning Unary

5    Critical Unary (non-likely except for power-off)

6    warning Binary

Training Level

2	Unknown

3	Untrained

4	Trained

5	Field Service

6	Management

7    No Intervention Required

Group Index

1	(Other)

2    (Unknown) N/A

3	hostResouurcesStorage

4	hostResourcesDevice

5	generalPrinter

6	Cover

7	Localization

8	Input

9	Output

10	Marker

13	Media Path

14	Channel

15	Interpreter

16	ConsoleDisplayBuffer

17	ConsoleLights