Subj:	Proposed Additions to the Printer MIB for Impression Limit

Control

From:	Tom Hastings

Date:	1/16/06

File:	implimit.doc

This paper is one of the action items from the December PWG meeting that Harry Lewis, Geoff Slater, and I had to propose the specification of three new objects for addition to the Printer MIB to allow hosts to control impression quotas.  The PWG agreed for us to take the three MIF attributes:

- “Printed Pages”

- “Pages to Print”

- “Status Time”

from Geoff’s white paper entitled: “PWG Proposed MIF & White Paper Security, Monitoring, and managing shared printers in the new economy” and propose specific objects to be added to the Printer MIB as the Printer MIB is progressed from proposed standard (RFC 1759) to draft standard.

PWG - There are two issues that are called out specifically.  Please send e-mail to the pmi DL about them and/or come prepared with answers to the upcoming PWG meeting in Miami on January 24-25.

# Description of Proposal

The proposal is to add three new objects as a new optional group entitled Impression Limit Control Group.  The three new objects are proposed to be added to the prtChannelTable, since each host that sends data to the printer needs to have its own control over impressions allowed, especially if the printer is switched between its channels dynamically.

PWG - ISSUE 01 - Can the three new objects be added to the existing prtChannelTable without causing the Printer MIB to have to re-start its six month requirement in the IETF standards track to becoming a draft standard?  Or should we add the three new objects as a new parallel table to the channel table, following SMIv2 quidelines for separate groups being in separate tables anyway?

The three new objects are:

- prtImpressionsPrinted - a read-only counter which counts the number of impressions since power up that have been printed through this channel.

PWG - ISSUE 02 - The prtImpressionsPrinted object is useful in its own right and is probably implemented in most printers.  Should we put the prtImpressionsPrinted object in the mandatory Channel group, instead of in the new optional Impressions Limit Control group so that prtImpressionsPrinted could be implemented without the other two objects? But doing so, might reset the six month IETF clock on making the Printer MIB a draft standard (see ISSUE 01 above)?

- prtImpressionLimit - a read-write limit which stalls the printer if the prtImpressionsPrinted reaches or exceeds this value.  The printer shall remain stalled until a new higher value or 0 is written to the prtImpressionLimit object or until the stall time exceeds the value specified by the prtStallTimeBeforeAbort object.  A value of 0 disables the feature so that the printer never stalls because of lack of impressions permitted on this channel.

- The printer generates the critical needImpressionLimitIncreased(1401) double change (nee binary change) event (and trap) when the printer stalls.

- The purpose of this object is to allow systems to make sure that a user is not able to print impressions for which he does not have the credit.  The feature is also designed so that malicious users cannot unplug the printer and avoid charges.

- prtStallTimeBeforeAbort - a read-write object which specifies the maximum length of time in ticks (same units as prt which is TimeTicks, i.e., 100ths of a second - see SMIv2 in RFC 1442) that this printer shall remain stalled due to the value of prtImpressionsPrinted meeting or exceeding a non-zero value of prtImpressionLimit.  Therefore, the printer cannot be stalled indefinitely waiting for a host to give permission for more impressions on this channel.

- The printer generates the non-critical impressionLimitExceededJobAbort(1402) single change (nee simple change) event (and trap) when the elapsed time of the printer stall exceeds the value of prtStallTimeBeforeAbort.  Then the printer is able to print other jobs on other channels (and on this channel after prtImpressionLimit is written with a larger value or 0).

# The proposed changes to the Printer MIB specification

The following changes are proposed to the indicated pages of the Printer MIB.  Only the pages that need changing are attached.

RFC 1759                      Printer MIB                     March 1995

interfaces are serial ports (with little or no protocol) and EtherNet

ports on which one might run InterNet IP, Novell IPX, etc.

2.2.10.  Channels

The channel sub-units identify the independent sources of print data

(here print data is the information that is used to construct printed

pages and may have both data and control aspects).  A printer may

have one or more channels. The channel sub-units are represented by

the Channel Group in the Model. Each channel is typically identified

by the electronic path and service protocol used to deliver print

data to the printer. A channel sub-unit may be independently enabled

(allowing print data to flow) or disabled (stopping the flow of print

data). It has a current Control Language which can be used to specify

which interpreter is to be used for the print data and to query and

change environment variables used by the interpreters (and SNMP).

There is also a default interpreter that is to be used if an

interpreter is not explicitly specified using the Control Language.

Channel sub-units are based on an underlying interface.  Optionally,

each channel may permit the host to limit the number of impressions

that may be printed over that channel before the printer stalls

waiting for the host to increase the limit.  This impression control

feature, if implemented, may also be disabled on a channel-by-channel

basis.

2.2.11.  Interpreters

The interpreter sub-units are responsible for the conversion of a

description of intended print instances into images that are to be

marked on the media. A printer may have one or more interpreters. The

interpreter sub-units are represented by the Interpreter Group in the

Model. Each interpreter is generally implemented with software

running on the System Controller sub-unit. The Interpreter Table has

one entry per interpreter where the interpreters include both Page

Description Language (PDL) Interpreters and Control Language

Interpreters.

2.2.12.  Console

Many printers have a console on the printer, the operator console,

that is used to display and modify the state of the printer.  The

console can be as simple as a few indicators and switches or as

complicated as full screen displays and keyboards. There can be at

most one such console.  This console sub-unit is represented by the

Console Group in the model.  Although most of the information

displayed there is also available in the state of the printer as

represented by the various Groups, it is useful to be able to query

and modify the operator console remotely.  For example, a management

application might like to display to its user the current message on

the operator console of the remote printer or the management

application user might like to modify the current message on the

operators console of the remote printer.  As another example, one

might have a remote application that puts up a pseudo console on a

Smith, Wright, Hastings, Zilles & Gyllenskog                   [Page 12]

RFC 1759                      Printer MIB                     March 1995

-- The Channel Table

--

-- The prtChannelTable represents the set of input data sources which

-- can provide print data to one or more of the interpreters

-- available on a printer

prtChannel OBJECT IDENTIFIER ::= { printmib 14 }

prtChannelTable OBJECT-TYPE

SYNTAX     SEQUENCE OF PrtChannelEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

""

::= { prtChannel 1 }

prtChannelEntry OBJECT-TYPE

SYNTAX     PrtChannelEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"Entries may exist in the table for each device

index who's device type is `printer'."

INDEX  { hrDeviceIndex, prtChannelIndex }

::= { prtChannelTable 1 }

PrtChannelEntry ::= SEQUENCE {

prtChannelIndex                     Integer32,

prtChannelType                      INTEGER,

prtChannelProtocolVersion           OCTET STRING,

prtChannelCurrentJobCntlLangIndex   Integer32,

prtChannelDefaultPageDescLangIndex  Integer32,

prtChannelState                     INTEGER,

prtChannelIfIndex                   Integer32,

prtChannelStatus                    SubUnitStatus,

-- next 3 are part of the optional Impression Limit Control group:

prtImpressionsPrinted               Counter32,

prtImpressionLimit                  Integer32,

prtStallTimeBeforeAbort             TimeTicks

}

prtChannelIndex OBJECT-TYPE

SYNTAX     Integer32

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"A unique value used by the printer to identify this data

channel.  Although these values may change due to a major

reconfiguration of the device (e.g. the addition of new data

channels to the printer), values are expected to remain

stable across successive printer power cycles."

Smith, Wright, Hastings, Zilles & Gyllenskog                   [Page 70]

RFC 1759                      Printer MIB                     March 1995

}

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The state of this print data channel.  The value determines

whether control information and print data is allowed through

this channel or not."

::= { prtChannelEntry 6 }

prtChannelIfIndex OBJECT-TYPE

SYNTAX     Integer32

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"The value of ifIndex (in the ifTable; see the interface

section of MIB-2/RFC 1213) which corresponds to this channel.

When more than one row of the ifTable is relevant, this is

the index of the row representing the topmost layer in the

interface hierarchy.  A value of zero indicates that no

interface is associated with this channel."

::= { prtChannelEntry 7 }

prtChannelStatus OBJECT-TYPE

SYNTAX     SubUnitStatus

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The current status of the channel."

::= { prtChannelEntry 8 }

-- The Impression Limit Control Group

--

-- This group is optional.  However, to claim conformance to this group,

-- it is necessary to implement every object in this group.

-- The impression limit control group is an optional group that contains

-- three objects that allow a host to limit the number of impressions

-- that the printer prints before the printer stalls awaiting the host

-- to increase the limit.  If the host fails to increase the limit after

-- a specified time, the printer aborts the current job, but is able to

-- print other jobs on other channels.  The impression control feature

-- maybe disabled on a channel-by-channel basis.

prtImpressionsPrinted OBJECT-TYPE

SYNTAX     Counter32

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"Counts the number of impressions printed through this channel

since powerup."

::= { prtChannelEntry 8 }

prtImpressionLimit OBJECT-TYPE

SYNTAX     Integer32

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object provides the limit of the number of impressions

that the printer shall print before stalling if the

prtImpressionsPrinted reaches or exceeds this value.  The

printer shall remain stalled until a new higher value or 0 is

written to the prtImpressionLimit object or until the stall time

exceeds the value specified by the prtStallTimeBeforeAbort

object.  A value of 0 disables the feature so that the printer

never stalls because of lack of impressions permitted on this

channel.

When the printer stalls, the printer generates the critical

needImpressionLimitIncreased(1401) double change event (and

trap).

Writing a value of 0 at any time shall disable the printer from

stalling for data being printed over this channel.  Writing a

non-zero value at any time into prtImpressionLimit shall resume

the comparing of the value of prtImpressionsPrinted with the

value of prtImpressionLimit written and stalling the printer

whenever the value of prtImpressionsPrinted meets or exceeds the

value of prtImpressionLimit.

The purpose of this object is to allow systems to make sure that

a user is not able to print impressions for which he does not

have the credit.  The feature is also designed so that malicious

users cannot unplug the printer and avoid charges."

::= { prtChannelEntry 9 }

prtStallTimeBeforeAbort OBJECT-TYPE

SYNTAX     TimeTicks

MAX-ACCESS read-write

STATUS     current

DESCRIPTION

"This object provides the maximum length of time in TimeTicks

(100ths of a second - see SMIv2) that this printer shall remain

stalled due to the value of prtImpressionsPrinted meeting or

exceeding a non-zero value of prtImpressionLimit.  Therefore,

the printer cannot be stalled indefinitely waiting for a host to

give permission for more impressions on this channel.  If the

value of prtImpressionLimit is not re-written with a value

larger than prtImpressionsPrinted (or a 0 value) before this

maximum time occurs, the printer shall abort the stalled job and

shall print other jobs on other channels, until the value of

prtImpressionLimit is re-written for this channel.

The printer generates the non-critical

impressionLimitExceededJobAbort(1402) single change event (and

trap) when the elapsed time of the printer stall exceeds the

value of prtStallTimeBeforeAbort.

-- The Interpreter Group

--

-- The interpreter sub-units are responsible for the conversion of a

-- description of intended print instances into images that are to be

-- marked on the media. A printer may have one or more interpreters. The

-- interpreter sub-units are represented by the Interpreter Group in the

-- Model. Each interpreter is generally implemented with software running

-- on the System Controller sub-unit. The Interpreter Table has one entry

-- per interpreter where the interpreters include both Page Description

-- Language (PDL) Interpreters and Control Language Interpreters.

--

-- Implementation of every object in this group is mandatory.

prtInterpreter OBJECT IDENTIFIER ::= { printmib 15 }

--       Interpreter Table

--

Smith, Wright, Hastings, Zilles & Gyllenskog                   [Page 73]

RFC 1759                      Printer MIB                     March 1995

configurationChange(7),

jam(8),

-- general Printer group

doorOpen(501),

doorClosed(502),

powerUp(503),

powerDown(504),

-- Input Group

inputMediaTrayMissing(801),

inputMediaSizeChange(802),

inputMediaWeightChange(803),

inputMediaTypeChange(804),

inputMediaColorChange(805),

inputMediaFormPartsChange(806),

inputMediaSupplyLow(807),

inputMediaSupplyEmpty(808),

-- Output Group

outputMediaTrayMissing(901),

outputMediaTrayAlmostFull(902),

outputMediaTrayFull(903),

-- Marker group

markerFuserUnderTemperature(1001),

markerFuserOverTemperature(1002),

-- Marker Supplies group

markerTonerEmpty(1101),

markerInkEmpty(1102),

markerPrintRibbonEmpty(1103),

markerTonerAlmostEmpty(1104),

markerInkAlmostEmpty(1105),

markerPrintRibbonAlmostEmpty(1106),

markerWasteTonerReceptacleAlmostFull(1107),

markerWasteInkReceptacleAlmostFull(1108),

markerWasteTonerReceptacleFull(1109),

markerWasteInkReceptacleFull(1110),

markerOpcLifeAlmostOver(1111),

markerOpcLifeOver(1112),

markerDeveloperAlmostEmpty(1113),

markerDeveloperEmpty(1114),

-- Media Path Device Group

mediaPathMediaTrayMissing(1301),

mediaPathMediaTrayAlmostFull(1302),

mediaPathMediaTrayFull(1303),

-- Channel Group

needImpressionLimitIncreased(1401),

impressionLimitExceededJobAbort(1402)

-- interpreter Group

interpreterMemoryIncrease(1501),

interpreterMemoryDecrease(1502),

interpreterCartridgeAdded(1503),

interpreterCartridgeDeleted(1504),

interpreterResourceAdded(1505),

Smith, Wright, Hastings, Zilles & Gyllenskog                   [Page 90]

RFC 1759                      Printer MIB                     March 1995

"It is conformant to implement just these two states in

this object.  Any additional states are optional."

OBJECT      prtConsoleOnTime

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

OBJECT      prtConsoleOffTime

MIN-ACCESS  read-only

DESCRIPTION

"It is conformant to implement this object as read-only."

-- the prtResponsiblePartyGroup, prtExtendedInputGroup,

-- prtInputMediaGroup, prtExtendedOutputGroup,

-- prtOutputDimensionsGroup, prtOutputFeaturesGroup,

-- prtMarkerSuppliesGroup, prtMarkerColorantGroup,

-- the prtAlertTimeGroup and the prtImpressionLimitControlGroup

are completely optional.

::= { prtMIBConformance 1 }

prtMIBGroups      OBJECT IDENTIFIER ::= { prtMIBConformance 2 }

prtGeneralGroup OBJECT-GROUP

OBJECTS { prtGeneralConfigChanges, prtGeneralCurrentLocalization,

prtGeneralReset, prtCoverDescription, prtCoverStatus,

prtLocalizationLanguage, prtLocalizationCountry,

prtLocalizationCharacterSet, prtStorageRefIndex,

prtDeviceRefIndex }

STATUS  current

DESCRIPTION

"The general printer group."

::= { prtMIBGroups 1 }

prtResponsiblePartyGroup OBJECT-GROUP

OBJECTS { prtGeneralCurrentOperator, prtGeneralServicePerson }

STATUS  current

DESCRIPTION

"The responsible party group contains contact information for

humans responsible for the printer."

::= { prtMIBGroups 2 }

prtInputGroup OBJECT-GROUP

OBJECTS { prtInputDefaultIndex, prtInputType, prtInputDimUnit,

prtInputMediaDimFeedDirDeclared,

prtInputMediaDimXFeedDirDeclared,

prtInputMediaDimFeedDirChosen,

prtInputMediaDimXFeedDirChosen, prtInputCapacityUnit,

prtInputMaxCapacity, prtInputCurrentLevel,

Smith, Wright, Hastings, Zilles & Gyllenskog                   [Page 93]

RFC 1759                      Printer MIB                     March 1995

STATUS  current

DESCRIPTION

"The alert table group."

::= { prtMIBGroups 17 }

prtAlertTimeGroup OBJECT-GROUP

OBJECTS { prtAlertTime }

STATUS  current

DESCRIPTION

"The alert time group."

::= { prtMIBGroups 18 }

prtImpressionLimitControlGroup OBJECT-GROUP

OBJECTS { prtImpressionsPrinted, prtImpressionLimit,

prtStallTimeBeforeAbort }

STATUS  current

DESCRIPTION

"The impression limit control group."

::= { prtMIBGroups 19 }

END

Smith, Wright, Hastings, Zilles & Gyllenskog                   [Page 97]