Printer Management Interface

Management Object Summary

By

Ronald L. Smith

Texas Instruments

email:	rlsmith@nb.ppd.ti.com
phone:	(817) 774-6151

Printer Component

To be supplied by DMTF to correspond to MIB II

DMTF MIF "Component ID: (From DMI Specification - Draft 4.2 - 12 January 1994)

1.	Manufacturer	String64	Name of the organization that produces this component.

2.	Product	String64	Name of this Printer

3.	Version	String64

4.	Serial Number	String64

5.	Installation	Date	Time and Date of the Last Installation

6.	Verify	Integer	Verification Level for this printer

"Asking for the 'Verify' attribute causes the component instrumentation code to perform checks to verify that the component is still in the system and working properly. It should return on of the following values:

Value	Meaning

0	An error occurred; Check status code

1	Component does not exist

2	Verify not supported

4	Component exists, functionally untested

5	Component exists, functionally unknown

6	Component exists, functionally no good

7	Component exists, functionally good

Printer Control Component Objects

| prtControl Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Status | Read-Only | Octet | PrinterStatus | Overall Printer Status |
| CounterUnits | Read-Only | Enum (1) | CounterUnits | Units that will be used to report counter values |
| LifeCount | Read-Only | Counter32 |  | Life of printer counter in CounterUnits |
| PowerOnCount | Read-Only | Counter32 |  | Number of units counted since power on in CounterUnits |

Notes:

| prtControl - Extended Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Reset | Write-Only | Integer |  | Writing a 1 simulates a power cycle |
| ResetSpecial | Write-Only | Enum (3) | ResetType | Writing a 1 causes a special reset |

Notes:

Input Component Objects - Mandatory

| prtInput Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer |  | ID of the Default Input Component |

Notes:

| prtInput Mandatory prtInputTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated input device |
| Type | Read-Only | Enum (2) | InputType |  |
| DimUnits | Read-Only | Enum (1) | PhyMediaUnit |  |
| CurDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CapacityUnit | Read-Only | Enum (1) | PhyMediaUnit |  |
| MaxCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurrentCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| Status | Read-Only | Octet | CompStatus | Current Status of Input Device |
| MediaName | Read-Write | String |  | "Hillary's Presidential Letterhead" |
| MediaSize | Read-Write | Enum (1) | MediaSize | ISO, JIS & North American media size identifiers |

Notes:

Input Component Objects - Optional

| prtInput - TypeExtended Optional prtInputTabelAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Name | Read-Writer | String |  | Name assigned to this device |
| VendorName | Read-Only | String |  |  |
| Model | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| SerialNumber | Read-Only | String |  |  |
| Description | Read-Only | String |  |  |
| Security | Read-Write | Enum (2) | YesNo-OnOff | Indication if the Device has some security associated with it. Note 1 |

Notes:

1.	Does there need to be an optional object that elaborates on the type of input security available?

| prtInput - DimExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| UnprintMarAcrossFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| UnprintMarFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| LeftMarginAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| RightMarginAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| TopMarginFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| BottomMarginFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |

Note:

The left justified objects are currently defined. I propose that they be eliminated and the italicized right justified objects be added. The current object do not define which margins are being defined.

| prtInput - MediaExtended Optional prtInputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MediaDimUnits | Read-Only | Enum (2) | PhyMediaUnits | Units used in the dimensions of media |
| MediaDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MediaDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MediaWeight | Read-Write | Integer32 | grams/meter2 | -1 = Other, -2 = Unknown |
| MediaType | Read-Write | String |  | ISO10175 (DPA) & ISO10180 (SPDL) Definitions |
| MediaColor | Read-Write | String |  | ISO10175 (DPA) & ISO10180 (SPDL) Definitions |
| MediaFormParts | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |

Notes:

Output Component Objects - Mandatory

| prtOutput Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer |  | ID of Default Output Device |

Notes:

| prtOutput Mandatory prtOutputTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated output device. |
| Type | Read-Only | Enum (2) | OutputType |  |
| CapacityUnit | Read-Only | Enum (1) | PhyMediaUnit |  |
| MaxCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurrentCapacity | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| Status | Read-Only | Octet | CompStatus |  |

Notes:

Output Component Objects - Optional

| prtOutput - TypeExtended Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Name | Read-Writer | String |  | Name assigned to this Device |
| VendorName | Read-Only | String |  |  |
| Model | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| SerialNumber | Read-Only | String |  |  |
| Description | Read-Only | String |  |  |
| Security | Read-Write | Enum (2) | YesNo-OnOff | Note 1, Note 2 |

Notes:

1.	I Propose that Device Security be moved from the Output Type Features subclass to be consistent with Input Device

2.	Does there need to be an optional object that elaborates on the type of output security available?

| prtOutput - DimExtended Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DimUnit | Read-Only | Enum (1) | PhyMediaUnit |  |
| CurDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| CurDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MinDimAcrossFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |
| MinDimFeedDir | Read-Write | Integer32 |  | -1 = Other, -2 = Unknown |

Notes:

| prtOutput - Features Optional prtOutputTableAugment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| StackingOrder | Read-Only | Enum (1) | StackingOrder |  |
| Bursting | Read-Write | Enum (1) | YesNo-OnOff |  |
| Decollating | Read-Write | Enum (1) | YesNo-OnOff |  |
| PageCollated | Read-Write | Enum (1) | YesNo-OnOff |  |
| Job Collated | Read-Write | Enum (1) | YesNo-OnOff |  |
| OffsetStacking | Read-Write | Enum (1) | YesNo-OnOff |  |
| Slipsheet | Read-Write | Enum (1) | YesNo-OnOff |  |
| DeviceSecurity | Read-Only | Enum (1) | YesNo-OnOff | Note1, Note 2 |

Notes

1.	I Propose that Device Security be moved to the Output Type Extened subclass to be consistent with Input Device

2.	Does there need to be an optional object that elaborates on the type of output security available?

| prtOutput - Associated Finisher Optional ?? |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Stitching | Read-Only | String |  |  |
| Folding | Read-Only | String |  |  |
| Binding | Read-Only | String |  |  |
| Punching | Read-Only | String |  |  |
| Trimming | Read-Only | String |  |  |
| Insertion | Read-Only | String |  |  |

Notes:

Finisher Component Objects - Optional

| prtFinisher Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | 0 = Finisher Not Used in Default Configuration |

Notes:

| prtFinisher Optional prtFinisherTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | String |  | Note 1 Value used by the printer to identify associated finisher device. |
| Type | Read-Only | String |  | Note 2 |
| NamedFinishing | Read-Only | String |  | Note 3 |
| ConusmableUnit | Read-Only | String |  | Note 4 |
| MaxConsumableCapacity | Read-Write | Integer32 |  | Note 4 -1 = Other, -2 = Unknown |
| CurrentConsumableCapacity | Read-Write | Integer32 |  | Note 4 -1 = Other, -2 = Unknown |
| Status | Read-Only | Octet | CompStat | Current Status of Device |

Notes: Above definition came from Printer Management Objects

1.	The ID object being a string is inconsistent with the rest of the objects. The ID should be an integer.

2.	The Type object being a string is inconsistent with the rest of the objects. The Type should be an enumeration with the enumeration's being of the value defined in FinisherType Enumeration Table.

3.	The NamedFinishing object being a string is inconsistent with other like objects. Should it be an Enumeration or a string?

4.	This approach limits each finisher to one "Consumable" and no provision has not been made for "receptacles" or "containers" that contain waste materials such as produced by a hole punch. Also it is possible that a particular finisher may have "supplies" and "waste".

Marker Component Objects - Mandatory

| prtMarker Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer32 |  | ID of Default Marker |

Notes:

| prtMarker Mandatory prtMarkerTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer32 |  | Value used by the printer to identify associated marker device. |
| MarkTech | Read-Only | Enum (1) | MarkingTech | Type of Marking Technology used for this making device |
| ProcessColorant | Read-Only | Integer32 |  | Number of Process Colors (-1 = Other, -2 = Unknown) |
| SpotColorant | Read-Only | Integer32 |  | Number of Spot Colors (-1 = Other, -2 = Unknown) |
| AddressabilityUnit | Read-Only | Enum (1) | AdressabilityUnit | Resolution Unit |
| AddressabilityFeedDir | Read-Only | Integer32 |  | Feed Resolution (-1 = Other, -2 = Unknown) |
| AddressabilityCrossFeedDir | Read-Only | Integer32 |  | CrossFeed Resolution (-1 = Other, -2 = Unknown) |
| Status | Read-Only | Octet | CompStatus | Current Status of Marker Component |

Notes:

| prtMarkerColorant Optional prtMarkerColorantTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MarkerID | Read-Only | Integer32 |  | ID of Marker that this colorant entry is associated with. |
| Role | Read-Only | Enum (1) | ColoranRole | Role played by this colorant |
| Order | Read-Only | Integer |  | Ordinal number of this colorant in the order in which colorants are imaged onto the medium within its role. |
| Value | Read-Only | String | ColorantValue | Name of the color. |

Notes:

Media Path Component Objects - Mandatory

| prtMediaPath Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| MaxSpeedPrintUnit | Read-Only | Enum (1) | SpeedPrintUnit | Numerator units of print speed |
| MaxSpeedTimeUnit | Read-Only | Enum (1) | SpeedTimeUnit | Denominator units of print speed |
| MaxSpeed | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| PhyMediaUnit | Read-Only | Enum (1) | PhyMediaUnit | Units of media size |
| MaxMediaFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MaxMediaAcrossFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MinMediaFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| MinMediaAcrossFeedDir | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |

Notes:

Media Path Device Component Objects - Optional

| prtMediaPathDevice Optional |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer |  | ID of Default Device 0 = Media Path Device is Not Used in Default Configuration |

Notes:

| prtMediaPathDevice Optional prtMediaPathDeviceTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| ID | Read-Only | Integer |  | Value used by the printer to identify associated media path device. |
| Type | Read-Only | Enum ( ) | MediaPathDeviceType |  |
| Status | Read-Only | Octet | CompStatus |  |

Notes:

Operator Console Objects

| prtOperatorConsole |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |

Interpreter Component Objects - Mandatory

| prtInterpreter Mandatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| DefaultID | Read-Write | Integer | IntpLangFam |  |

Notes:

| prtInterpreter Mandatory prtInterpreterTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| LangFamily | Read-Only | Integer | IntpLangFam |  |
| LangLevel | Read-Only | String |  |  |
| LangVersion | Read-Only | String |  |  |
| Desciption | Read-Only | String |  |  |
| Version | Read-Only | String |  |  |
| FeedAddressability | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| CrossFeedAddressability | Read-Only | Integer32 |  | -1 = Other, -2 = Unknown |
| DefaultOrientation | Read-Write | Enum (2) | MediaOrientation |  |

Notes:

Critical Alerts Component Object - Mandatory

| prtCriticalAlert Mandatory prtCriticalAlertTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Index | Read-Only | Enum ( ) |  |  |
| SeverityLevel | Read-Only | Enum ( ) |  |  |
| InfoClass | Read-Only | Enum ( ) |  |  |
| Component | Read-Only | Enum ( ) |  |  |
| ComponentID | Read-Only | Integer |  |  |
| ComponentLocation | Read-Only | Integer |  |  |
| Code | Read-Only | Enum ( ) |  |  |
| Description | Read-Only | String |  |  |

Notes:

Critical Alerts Component Object - Mandatory

| prtNonCriticalAlert Mandatory prtNonCriticalAlertTable |  |  |  |  |
| --- | --- | --- | --- | --- |
| Object Name | Access | Type | Value | Description |
| Index | Read-Only | Enum ( ) |  |  |
| SeverityLevel | Read-Only | Enum ( ) |  |  |
| InfoClass | Read-Only | Enum ( ) |  |  |
| Component | Read-Only | Enum ( ) |  |  |
| ComponentID | Read-Only | Integer |  |  |
| ComponentLocation | Read-Only | Integer |  |  |
| Code | Read-Only | Enum ( ) |  |  |
| Description | Read-Only | String |  |  |

Notes:

Component Status Octet

| Printer Status Bit Meaning 0 NonCritical Alert Active 1 Critical Alert Active 2 On Request 3 Unavailable 4 Busy / Temporarily Unabailable 5 Line Status Bit 0 6 Line Status Bit 1 Bit 6 Bit 7 Meaning 0 0 On Line 0 1 Pending On Line 1 0 Pending Off Line 1 1 Off Line 7 Not Used | Component Status Bit Meaning 0 NonCritical Alert Active 1 Critical Alert Active 2 On Request 3 Unavailable 4 Busy / Temporarily Unabailable 5 Not Used 6 Not Used 7 Not Used |
| --- | --- |

PMI Enumerations - Units

| Addressabililty Unit 1 Other 2 Unknown 3 Inches 4 Centimeters | Physical Media Unit 1 Other 2 Unknown 3 Inches 4 Millimeters 5 Sheets | Counter Unit 1 Other 2 Unknown 3 Characters 4 Lines 5 Impressions 6 Inches 7 Feet 8 Millimeters 9 Sheets 10 Seconds 11 Minutes 12 Hours |
| --- | --- | --- |
| Speed Print Unit 1 Other 2 Unknown 3 Characters 4 Lines 5 Impressions 6 Inches 7 Feet 8 Millimeters 9 Meters 10 Sheets | Speed Time Unit 1 Other 2 Unknown 3 Seconds 4 Minutes 5 Hours | Finisher Consumable Unit 1 Other 2 Unknown |

PMI Enumerations - Device Types

| Input Type 1 Other 2 Unknown 3 Sheet-feed-auto-removable-tray 4 sheet-feed-auto-non removable tray 5 Sheet Feed Manual 6 continuous-roll 7 continuous-fan-fold | Output Type 1 Other 2 Unknown 3 Removable Bin 4 UnRemovable Bin 5 Continuous Roll Device 6 Mail Box | Media Path Device Type 1 Other 2 Unknown 3 Long Edge Binding 4 Short Edge Binding |
| --- | --- | --- |
| Finisher Type 1 Other 2 Unknown 3 Stitching 4 Binding 5 Trimming 6 Die-cutting 7 Punching 8 Perforating 9 Slitting 10 Inserting 11 Covers 12 Folding | Interface Type 1 Other 2 Unknown 3 RS-232 4 IEEE1284 Parallel 5 Centronics Parallel 6 Local Talk 7 Network 8 SCSI 9 Coax 10 Twinax 11 Fax 12 IEEE 1394 | Interpreter Language Family 1 Other 2 Unknown 3 PCL 4 HPGL 5 PJL 6 PostScript 7 PPDS 8 EscapeP 9 Epson 10 DDIF 11 Interpress 12 ISO6429 13 LinePrinter 14 MODCA 15 ReGIS 16 SCS 17 SPDL 18 Tek4014 19 PDS 20 IGP 21 Code-V 22 DSC-DSE 23 WPS 24 LN03 25 CCITT 26 QUIC |
| Marking Technology 1 Other 2 Unknown 3 Electophotographic:LED 4 Electrophotographic:Laser 5 Electrophotographic:Other 6 Impact:Moving_Head:Dot_Matrix:9pin 7 Impact:Moving_Head:Dot_Matrix:24pin 8 Impact:Moving_Head:Dot_Matrix:other 9 Impact:Moving_Head:Fully_Formed 10 Impact:band 11 Impact:other 12 Inkjet:Aqueous 13 Inkjet:Solid 14 Inkjet:Other 15 Pen 16 Thermal:Transfer 17 Thermal:Sensitive 18 Thermal:Diffusion 19 Thermal:Other 20 Electroerosion 21 Electrostatic 22 Photographic:Microfiche 23 Photographic:Imagesetter 24 Photographic:Other 25 Ion-deposition 26 E-beam 27 Typesetter |  |  |

PMI Enumerations - Misc

| Media Orientation 1 Other 2 Unknown 3 Portrait 4 Reverse Portrait 5 Landscape 6 Reverse Landscape | Media Color 1 Other 2 Unknown 3 White 4 Pink 5 Yellow 6 Buff 7 Goldenrod 8 Blue 9 Green 10 Transparent | Line Status 1 Other 2 Unknown 3 On Line 4 Pending On Line 5 Off Line 6 Pending Off Line |
| --- | --- | --- |
| Named Finshing 1 Other 2 Unknown 3 Staple 4 Staple Top Left 5 Staple Bottom Left 6 Stable Top Right 7 Staple Bottom Right 8 Saddle Stitch 9 Edge Stitch 10 Punch 11 Cover 12 Bind | MediaSize 1. Other 2. Unknown 3. Use Dimensions not Enumeration 4. -- 5. -- | StackingOrder 1. Other 2. Unknown 3. Face-Up 4. Face-Down |
| YesNo -OnOff 1. Other 2. Unknown 3. Yes - On 4. No - Off | ResetType 1. Other 2. Unknown 3. Reset NVRAM 4. Reset to Factory Defaults |  |
| ColorantRole 1. Other 2. Unknown 3. Process 4. Spot | ColorantValue - String ASCII representation of the integer asociated with the folloing names. 1. Other 2. Unknown 3. White 4. Red 5. Green 6. Blue 7. Cyan 8. Magenta 9. Yellow 10. Black Additional names using ISO9070 naming convention. |  |

PMI Enumerations - Alert Codes

| Security Level Alert Code 1 Other 2 Unknown 3 Critical 4 Severly Critical 5 Mild 6 Severly Mild | Alert Class Code 1 Other 2 Unknown 3 Management 4 Technical 5 Operator | Alert Component Code 1 Other 2 Unknown 3 Input 4 Output 5 Finisher 6 Media Path 7 Media Path Device 8 Marker 9 System Controller 10 Interface 11 Interpreter 12 Operator Console |
| --- | --- | --- |
| Input Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Size Change 11 Media Weight Change 12 Media Type Change 13 Media Color Change 14 Media Form Parts Change 15 Media Supply Low 16 Media Supply Empty | Output Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Tray Almost Full 11 Media Tray Full | Marker Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Toner Empty 10 Ink Empty 11 Print Ribbon Empty 12 Toner Almost Empty 13 Ink Almost Empty 14 Print Ribbon Almost Empty 15 Waste Toner Receptical Almost Full 16 Waste Ink Receptical Almost Full 17 Waste Toner Receptical Full 18 Waste Ink Receptical Ful 19 Fuser Under Temperature 20 Fuser Over Temperature 21 OPC LIfe Almost Over 22 OPC Life Over 23 Developer Almost Empty 24 Developer Empty |
| Media Path Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam | Media Path Device Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Media Tray Almost Full 11 Media Tray Full | Finisher Alert Code 1 Other 2 Unknown 3 Cover Open 4 Cover Closed 5 Interlock Open 6 Interlock Closed 7 Configuration Change 8 Jam 9 Media Tray Missing 10 Supply Empty 11 Supply Low 12 Receptical Almost Full 13 Receptical Full |
| Interpreter Alert Code 1 Other 2 Unknown 3 Configuration Change 4 Memory Increase 5 Memory Decrease 6 Cartridge Added 7 Cartridge Deleted 8 Resource Added 9 Resource Deleted | Interface Alert Code 1 Other 2 Unknown 3 Configuration Change |  |