PRINTER MANAGEMENT OBJECTS

PRINTER CLASS

Draft 0.3

F.D. Wright

Lexmark International, Inc.

July 30, 2026

# ASSUMPTIONS:

- The DMTF will define a "SYSTEM" Group that corresponds to the System group within MIB-II.  This will be available for all devices.  It is therefore not included explicitly in this document.

- Enumerations will always provide for Other (1) and Unknown (2).

- Adding additional enumerations may be handled in three ways:

1) Enumerations changed by altering this base document or RFC.

2) Enumerations changed by registering with the IANA with the concurrence of the working group.

3) Enumerations changed by registering with the IANA with no review by the working group.

These are referred to within each object, as required, as "enum type."

# PRINTER CLASS

Start Group

Start Attribute

Name=prtMarkTech

Access=Read-only

Description="This object described the marking technology of this 					device.  It has an enum control type of 1."

Type=	start enum

Other = 1

Unknown = 2

Electrophotographic:LED = 3

Electrophotographic:Laser = 4

Electrophotographic:Other = 5

Impact:Moving_Head:Dot_Matrix:9pin = 6

Impact:Moving_Head:Dot_Matrix:24pin = 7

Impact:Moving_Head:Dot_Matrix:other = 8

Impact:Moving_Head:Fully_Formed = 9

Impact:band = 10

Impact:other = 11

Inkjet:Aqueous = 12

Inkjet:Solid = 13

Inkjet:Other = 14

Pen = 15

Thermal:Transfer = 16

Thermal:Sensitive = 17

Thermal:Diffusion = 18

Thermal:Other= 19

Electroerosion = 20

Electrostatic = 21

Photographic:Microfiche = 22

Photographic:Imagesetter = 23

Photographic:Other = 24

Ion-depositon = 24

E-beam = 26

Typesetter = 27

end enum

End Attribute

Start Attribute

Name=prtProcessColorant

Access=Read-only

Type=Integer

Description="This objects contains the number of process 						colors supported by the marking engine.  A process color 				of 1 implies monochrome.  The value of this object and 					prtSpotColorant cannot both be 0.  A value of -1 implies 				'other' or 'infinite' while a value of -2 implies 'unknown'."

End Attribute

Start Attribute

Name=prtSpotColorant

Access=Read-only

Type=Integer

Description="This objects contains the number of spot 							colors supported by the marking engine.  The value of 					this object and prtProcessColorant cannot both be 0.

A value of -1 implies 'other' or 'infinite' while a value

of -2 implies 'unknown'."

End Attribute

Start Attribute

Name=prtMaxSpeedPrintUnits

Access=Read-only

Description="This object describes the print units used in specifying

the printing speed of the device.  This object is

a part of the numerator.  It has an enum control of type 1.

Type= start enum

Other = 1

Unknown = 2

Characters = 3

Lines = 4

Impressions = 5

Inches = 6

Feet = 7

Millimeters = 8

Meters = 9

Sheets = 10

end enum

End Attribute

Start Attribute

Name=prtMaxSpeedTimeUnits

Access=Read-only

Description="This object describes the time units used in specifying

the printing speed of the device.  This object is

a part of the denominator.  It has an enum control of

type 1."

Type=	start enum

other = 1

unknown = 2

seconds = 3

minutes = 4

hours = 5

end enum

End Attribute

Start Attribute

Name=prtMaxSpeed

Access=Read-only

Type=Integer

Description="Maximum printing speed of the engine as specified 					by PrtMaxSpeedTimeUnits.  A value of -1 implies 'other' or 				'unknown' while a value of -2 implies 'unknown'."

End Attribute

Start Attribute

Name-prtAddressabilityUnits

Access=Read-only

Description="This object contains the units of the denominator of 					Horizontal and Vertical addressability.  It has an enum				control of type 1."

Type=	start enum

other = 1

unknown = 2

inches = 3

centimeters = 4

end enum

End Attribute

Start Attribute

Name=prtAddressabilityFeedDirection

Access=Read-only

Type=Integer

Description="Addressability of the engine in the feed direction per 					units specified by prtAddressabilityUnits.  A value

of -1 implies 'other' or 'infinite' while a value of -2

implies 'unknown'."

End Attribute

Start Attribute

Name=prtAddressabilityCrossFeedDirection

Access=Read-only

Type=Integer

Description="Addressability of the engine in the cross feed direction 					per units specified by prtAddressabilityUnits.  A value

of -1 implies 'other' or 'infinite' while a value of -2					implies 'unknown'."

End Attribute

Start Attribute

Name=prtPhyMediaUnits

Access=Read-only

Description="This object describes the units used in specifying

the media size.  This object has an enum control of

type 1."

Type=	start enum

other = 1

unknown = 2

0.001 inches = 3

0.01 millimeters = 4

end enum

End Attribute

Start Attribute

Name=prtMaxPhyMediaFeedDir

Access=Read-only

Type=Integer

Description="Maximum physical media size in the feed direction

expressed in units specified by prtPhyMediaUnits.

A value of -1 implies 'other' or 'infinite' while a value of -				-2 implies 'unknown'.  A value of 0 implies unlimited."

End Attribute

Start Attribute

Name=prtMinPhyMediaFeedDir

Access=Read-only

Type=Integer

Description="Minimum physical media size in the feed direction

expressed in units specified by prtPhyMediaUnits.

A value of -1 implies 'other' while a value of -2 implies

'unknown'."

End Attribute

Start Attribute

Name=prtMaxPhyMediaAcrossFeedDir

Access=Read-only

Type=Integer

Description="Maximum physical media size across the feed direction

expressed in units specified by prtPhyMediaUnits.

A value of -2 implies 'unknown'. "

End Attribute

Start Attribute

Name=prtMinPhyMediaAcrossFeedDir

Access=Read-only

Type=Integer

Description="Minimum physical media size across the feed direction

expressed in units specified by prtPhyMediaUnits.  A

value of -2 implies 'unknown'."

End Attribute

Start Attribute

Name=prtDuplexAvailable

Access=Read-only

Description="This object indicated whether the print engine is

capable of printing on both sides of the media.  This

process can be implemented by single or multiple

passes of the media through the engine.  This object has

an enum control of type 1."

Type=	start enum

other = 1

unknown = 2

yes = 3

no = 4

end enum

End Attribute

Start Attribute

Name=prtCounterUnits

Access=Read-only

Description="This objects contains the units that will be used by

the printer when reporting counter values.  The time

units are provided for a device like a strip recorder

that does not or cannot track the physical dimensions

of the media and does not use characters, lines

or sheets.  This object has an enum control of type 1."

Type=	start enum

Other = 1

Unknown = 2

Characters = 3

Lines = 4

Impressions = 5

Inches = 6

Feet = 7

Millimeters = 8

Meters = 9

Sheets = 10

seconds = 11

minutes = 12

hours = 13

end enum

End Attribute

Start Attribute

Name=prtLifeCounter

Access=Read-only

Type=Counter

Description="Life of printer counter of the number of units as 					specified by prtCounterUnits."

End Attribute

Start Attribute

Name=prtPowerOnCounter

Access=Read-only

Type=Counter

Description="This counter contains the count of the number of 					units counted since the equipment was most

recently powered on using units as specified by 						prtCounterUnits."

End Attribute

Start Attribute

Name=prtRAMSize

Access=Read-only

Type=Integer

Description="Number of k-bytes (1024)  present in the printer."

End Attribute

Start Attribute

Name=hrDeviceStatus

Description="This attribute is taken from the host MIB."

End Attribute

Start Attribute

Name=hrPrinterStatus

Description="This attribute is taken from the host MIB."

End Attribute

Start Attribute

Name=prtDetailStatus

Access=Read-only

Type=Displaystring

Description="Detailed description of the printer's state as follows:

*****TBD*****

Note:  This item was deferred at meeting #2 and at meeting 				#3."

End Attribute

# PRINTER CLASS, COLORANT DETAIL SUBCLASS (optional)

Start Group

Name="Printer Colorant Details"

Start Table

Name="PrinterColorantsTable"

Start Attribute

Name=prtColorantRole

Access=Read-only

Type=	start enum

other = 1

unknown = 2

process = 3

spot = 4

end enum

Description="Role played by this colorant.  This enum is 						of control type 1.

End Attribute

Start Attribute

Name=prtColorantOrder

Access=Read-only

Type=Integer

Description="The ordinal number of this colorant in the 						order in which colorants are imaged onto the

medium within its role."

End Attribute

Start Attribute

Name=prtColorantValue

Access=Read-only

Type=displaystring

Description="This identifies the name of the color using

the following names.  Additional names may

be added using ISO9070 naming convention.

Predefined names below are returned

as the ASCII representation of the integer

associated with the name.

- Other (1)

- Unknown (2)

- White (3)

- Red (4)

- Green (5)

- Blue (6)

- Cyan (7)

- Magenta (8)

- Yellow (9)

- Black (10)"

End Attribute

Start Attribute

Name=prtColorantTonality

Access=Read-only

Type=Integer

Description="The object represents the distinct levels of						tonality realizable by the engine.  This does not

cover the number of levels of tnoal difference

that an interpreter can obtain by techniques

such as halftoning.  This object must be at

least 2."

End Attribute

End Table

End Group

# PRINTER CLASS, CONTROL SUBCLASS (optional)

Start Group

Name="Printer Controls"

Start Attribute

Name=prtctrlReset

Access=Write-only

Type=Integer

Description="Writing a 1 instructs the printer to reset 							simulating a power cycle.  Each manufacturer

will determine what events or activities are

to occur before the reset occurs."

End Attribute

Start Attribute

Name=prtctrlResetSpecial

Access=Write-only

Description="Writing to this object instructs the printer to

reset in special ways.  Each manufacturer

will determine what events or activities are

to occur before the reset occurs.  This object has

enum control of type 3."

Type=	start enum

Other = 1

Unknown = 2

Reset NVRAM = 3

Reset to factory defaults = 4

end enum

End Attribute

Start Attribute

Name=prtDuplexDefault

Access=Read-write

Description="This object indicates the current duplexing

default.  The state of this object applies to

to all input, all interpreters, etc. unless

overriden by other control.  This object has

enum control of type 1."

Type=	start enum

Other = 1

Unknown = 2

Long Edge Binding = 3

Short Edge Binding = 4

None = 5

End Attribute

Start Attribute

Name=prtDefaultOrient

Access=Read-write

Description="This object indicates the current orientation

default.  The state of this object applies to

to all input, all interpreters, etc. unless

overriden by other control.  This object has

enum control of type 1."

Type=	start enum

Other = 1

Unknown = 2

Portrait = 3

Reverse Portrait = 4

Landscape = 5

Reverse Landscape = 6

end enum

End Attribute

Start Attribute

Name=prtctrlLineStatus

Access=Read-write

Description="This object indicates the current on-line/offline

state of the printer.  This object is of enum

control type 1."

Type=	start enum

Other = 1

Unknown = 2

On-line = 3

Pending on-line = 4

Off-line = 5

Pending off-line = 6

end enum

End Attribute

Start Attribute

Name=prtctrlControlPanelLanguage

Access=Read-write

Type=Displaystring

Description="Reading returns the current language, writing

sets it.  Language values read or written are as

defined in ISO 1024.

***

PENDING CONFIRMATION OF ISO1024

OR SOME OTHER STANDARD FOR

LANGUAGE NAMES.

*** "

End Attribute

End Group

End Group