Multifunction Peripheral Association

Technical Contribution To Printer Working Group

Date:

Ref:	Port Mapping for IEEE-1284 and other interfaces

Source:	Raymond Lutz (Cognisys, Inc. Representing MFPA)

V:619-447-3246	F:619-447-6872	E: <raylutz@cognisys.com>

Title:	Service Triplet Assignment Guidelines

File:	FN:input.doc; DN:MFPA-96-002

# Abstract

This document provides guidelines for assigning Service Triplets for use by drivers and other intelligent software to determine the sort of services provided by a device. The document MFPA-95-037 describes a method of describing the Service available in the device using a Service Triplet for the IEEE-1284 interface. The ServiceRegistry, ServiceFamily, and ServiceVersion comprise the ServiceTriplet, which uniquely and unambiguously identifies the service. It is a requirement that the Access Protocol (i.e. raw stream, Remote procedure call, etc.) be implied by the ServiceTriplet, and so it is not specified separately. This document forms the basis for numerical assignments of the values in the ServiceTriplet, and provides guidelines for assignments and harmonization of preexisting service registries.

# Introduction

A peripheral or other intelligent device is connected to a host computer. The peripheral will provide a set of services to the host computer. Also, services may exist in the host computer which are usable by the attached peripheral devices. Between the devices, a communication channel of some type is provided. This channel must have some means of identifying the services that are available to the other. The method of communicating this information varies on different channels, but the need for a consistent syntax and semantics to describe the services is needed. Historically, each vendor provides this information in a vendor specific way, such as Manufacturer, Model, Version designations. Because of the need to allow dial-up or other access by drivers and entities that are not local and may not have access to the specific Manufacturer/Model information, the need for the approach as described in this document became apparent.

# Service Triplet

The concept of a service provider or server will be used in this development. A given service will be provided on a given "port" or logical channel of the interface. For an interface which does not have multiple logical channels, there will be a single service provider (server). For those with multiple channels, there may be more than one server, one for each logical channel. A given server may provide multiple capabilities. Each capability is specified as a separate Service Triplet. Multiple Service Triplets may be specified to fully characterize the server.

The ServiceRegistry, ServiceFamily, and ServiceVersion comprise the ServiceTriplet. This triplet exactly identifies the access protocol, and command set, language, or RPC (Remote Procedure Call) function set available as a capability of that server. Because there may be more than one capability of the server, several ServiceTriplets may be specified if the syntactical and semantic elements of one service is a proper subset of another, such that the interaction with either service is supported with no automatic switching required. This is commonly the case for services which have been upgraded so as to be backward compatible with older versions of the same service.

Also, if automatic switching is supported between services where the syntactical and semantic elements of one is not a proper subset of the other, then these may also be specified for the same Server.

The ServiceTriplet may inexactly identify a command set by leaving the ServiceVersion as "Not Applicable" only if the service identified by the other two components of the triplet has syntactical and semantic facilities that can be used to identify any version variations of that service. It is recommended, however,  that the various versions which are selectable by that service also be listed under the same ServiceTag to allow more up-front selection by the Host without resorting to deeper inquiry utilizing the specific syntax of that service. It is the goal of this recommendation to maintain knowledge of the syntax and semantics of the services as unnecessary to perform the service selection process.

## Method of Identifying Services

The method of specifying the capabilities of the service will vary based on the physical channel. The IEEE-1284 method will be used here to allow complete specification of the method. That method uses a ServiceTag so that more than one service triplet may be assigned to a given ServiceTag, and there is only one ServiceTag per server type. The expression of the service capabilities provided by a given ServiceTag is provided by the SRVC statement. The mapping of these services to logical channel addresses is performed by the SCKT statement. The exact syntax of the SRVC and SCKT statements is not defined here. Further information can be found in the referenced document.

## ServiceRegistry

The ServiceRegistry identifies a specific registry which provides the ServiceFamily and ServiceVersion values. This allows various registries to be utilized for various functions within the device, yet not require passing a variable-length "OBJECT IDENTIFIER" prefix. This document will therefore become the "Registry of Registries".

0	= Reserved

1	= Unknown

2	= Not Applicable

3	= Manufacturer Specific

4	= This document, see below

5	= (PWG) RFC-1759 PrtInterpreterFamily and IANA extensions

6	= (AIIM) Scanner

7	= (TIA) Facsimile

8	= (TIA) Data / Simultaneous Voice and Data / Voice

9	= File Services

10	= Database Services

>10	= Reserved for Standardization

## ServiceFamily

The Service Family should provide a Family of Services which have the same syntax and semantics or are otherwise automatic switching between different semantics and syntaxes. The ServiceFamily may be all that is necessary to completely specify a given service capability, and if that is the case, the ServiceVersion shall be specified as '02', "Not Applicable".

## ServiceVersion

The service version specifies variations within a given ServiceFamily. If an entity is unfamiliar with a specific version within a known ServiceFamily, that entity may wish to attempt to work with the responder within the known parameters of the ServiceFamily. This is an important benefit derived by breaking the identity of a service into a Family and Version.

The following ServiceVersion values are standard for all ServiceFamily registries.

0		= Reserved

1		= Unknown

2		= Not Applicable - Family set can self-identify version generations.

For other values, see the registry as specified by the ServiceRegistry parameter.

# Defined ServiceTriplet Values

The following ServiceTriplet values are defined by this document:

4,1,1		= Unknown

4,2,1		= Echo

4,3,2		= Internal Session-Level Port Manager

4,4,2		= MLC Command Processor

# Harmonization with RFC-1759 PrtInterpreterFamilyTable

RFC-1759 "Printer MIB" provides a table of values which define services provided by a printer. In this case, the services are limited to the definition of the Page Description Language Interpreter or Job Control Language Interpreter. In some cases, knowledge of the Interpreter Language is sufficient to allow determination of the other service capabilities that can be provided by the device. In many cases, due to the historical unidirectional channel utilized by printers, it is not always possible to determine the full set of capabilities from this information.

The prtInterpreterLangTable consists of entries describing a given interpreter available on the printer. The attribute prtInterpreterLangFamily is a controlled ("Type 2") enumeration, so that each addition requires committee review. It is defined as "the family name of a Page Description Language (PDL) or control language which this interpreter in the printer can interpret or emulate."

The prtInterpreterLangLevel allows further definition of the interpreter language. Unfortunately, this value is not controlled and requires no review. There are no firm rules about the contents of this attribute. It is defined as "The level of the language which this interpreter is interpreting or emulating. This might contain a value like '5e' for an interpreter which is emulating level 5e of the PCL language. It might contain '2' for an interpreter which is emulating level 2 of the PostScript language. Similarly it might contain '2' for an interpreter which is emulating level 2 of the HPGL language."

It is desired to allow drivers and other software to make a determination of the capabilities of the device without needing additional "smarts" regarding the backward compatibility of each version. Therefore, it is desired to define the prtInterpreterLangLevel further to obtain this precision. The prtInterpreterLangFamily will be used to express both the ServiceFamily and ServiceVersion in the following way:

1) The existing values will be multiplied by 100 to form the new PrtInterpreterLangFamily values. Values less than 100 will be deprecated.

2) The ServiceVersion value will be a value less than 100 and will be added to the PrtInterpreterLangFamily base as derived in step (1).

3) When a service is declared, it is preferred that all compatible services capabilities be described separately such that a user of the information need not keep or know any other compatibility information to determine if the service will perform the desired function. (This is in contrast to the current intention that only one PrtInterpreterLangEntry be provided for each interpreter, and compatibilities with older versions of that Interpreter would only be implied. Instead, multiple entries may exist to explicitly describe various interpreter capabilities.)

The result is a flat list of identifiers which are structured according to the family and level. This list is summarized below. (This list is summarized from the list in the RFC-1759 document and does not include all the appropriate trademark and reference information. Please see that document for more detail.) Items that deserve special note are highlighted.

prtInterpreterLangFamily OBJECT-TYPE

-- This value is a type 2 enumeration

SYNTAX     INTEGER {

other(1),

langPCL3(303),     -- PCL3

langPCL4(304),     -- PCL4

langPCL5(305),     -- PCL5

langPCL5e(306),    -- PCL5e

langHPGL(403),     -- HPGL

langHPGL2(404),    -- HPGL/2

langPJL(502),      -- Peripheral Job Language. Can provide

capabilities internally.

langPS1(603),      -- PostScript Level I Language

langPS2(604),      -- PostScript Level II Language

langPSPrinter(4202),

-- The PostScript Language used for

-- control (with any PDLs)

langIPDS(702),     -- Intelligent Printer Data Stream

langPPDS(803),     -- IBM Personal Printer Data Stream.

-- Originally called IBM ASCII, the name

-- was changed to PPDS when the Laser

-- Printer was introduced in 1989.

langEscapeP(903),

langEpson(1003),

langDDIF(1103),    -- Digital Document Interchange Format

langInterpress(1203),

langISO6429(1303), -- ISO 6429.

langLineData(1403),-- line-data See ISO 10175 Document

-- Printing Application (DPA)

langMODCA(1503),   -- Mixed Object Document Content

-- Architecture

langREGIS(1603),   -- Remote Graphics Instruction Set,

langSCS(1703),     -- SNA Character String

langSPDL(1803),    -- ISO 10180 Standard Page Description

-- Language

-- ISO Standard

langTEK4014(1903),

langPDS(2003),

langIGP(2103),

langCodeV(2203),   -- Magnum Code-V, Image and printer control

langDSCDSE(2303),  -- DSC-DSE:  Data Stream Compatible and

langWPS(2403),     -- Windows Printing System, Resource based

langLN03(2503),    -- Early DEC-PPL3, Digital Equipment Corp.

langCCITT_T4(2603),-- Group 3 facsimile encoding

-- (See ITU-T T.4)

-- NOTE: Page breaks not defined.

langCCITT_T4(2604),-- Group 4 facsimile encoding

-- (See ITU-T T.6)

-- NOTE: Page breaks not defined.

langQUIC(2703),    -- QUIC (Quality Information Code), Page

langCPAP(2803),    -- Common Printer Access Protocol

langDecPPL(2903),  -- Digital ANSI-Compliant Printing Protocol

langSimpleText(3003),

-- simple-text:  character coded data,

-- including NUL, CR , LF, HT, and FF

-- control characters.

langNPAP(3102),    -- Network Printer Alliance Protocol

-- IEEE 1284.1

langDOC(3203),     -- Document Option Commands

langimPress(3303), -- imPRESS, Page description language

langPinwriter(3403),

-- 24 wire dot matrix printer

langNPDL(3503),    -- Page printer  for Japanese market.

langNEC201PL(3603),-- Serial printer language used in the

-- Japanese market.

langAutomatic(37), -- (Deprecate)

langPages(3803),   -- Page printer Advanced Graphic Escape Set

langLIPS(3903),    -- LBP Image Processing System

langTIFF(4003),    -- Tagged Image File Format (Aldus) V 5

langTIFF(4004),    -- Tagged Image File Format (Aldus) V 6

langDiagnostic(4103),

-- A hex dump of the input to the

-- interpreter

langCaPSL(4303),   -- Canon Print Systems Language

langEXCL(4403),    -- Extended Command Language

langLCDS(4503),    -- Line Conditioned Data Stream

langXES(4603)      -- Xerox Escape Sequences

}

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The family name of a Page Description Language (PDL) or

control language which this interpreter in the printer can

interpret or emulate.  This type 2 list of enumerations

requires review before additional entries are made. The portion

of the value greater than 100 provides the LangFamily

information. The portion less than 100 provides the LangLevel

information."

::= { prtInterpreterEntry 2 }

The prtInterpreterLangLevel will remain as a user-readable interpretation of the Level (ServiceVersion) enumeration. This is used by management stations for direct display to the user. It is generally not used by drivers and other software wishing to gain capability information.

prtInterpreterLangLevel OBJECT-TYPE

SYNTAX     OCTET STRING (SIZE(0..31))

MAX-ACCESS read-only

STATUS     current

DESCRIPTION

"The level of the language which this interpreter is

interpreting or emulating.  This might contain a value like

'5e' for an interpreter which is emulating level 5e of the PCL

language.  It might contain '2' for an interpreter which is

emulating level 2 of the PostScript language.  Similarly it

might contain '2' for an interpreter which is emulating level

2 of the HPGL language."

::= { prtInterpreterEntry 3 }

# Summary

This document includes a proposal to the Printer Working Group for changes to the PrtInterpreterLangFamily enumeration, including change of existing values to values greater than 100 and deprecation of the old values. Additional entries would be made to allow for know versions of the popular languages. This single enumeration would be maintained in the normal fashion (through IANA). Also, it is suggested that managed printers respond not with a single service, but will all services provided by that printer. For example, if a printer was a PCL5e printer, it would respond with all of the following:

langPCL3(303),     -- PCL3

langPCL4(304),     -- PCL4

langPCL5(305),     -- PCL5

langPCL5e(306),    -- PCL5e

langHPGL(403),     -- HPGL

langHPGL2(404),    -- HPGL/2

If the capabilities of the language is inherently available internal to that language, such as with PJL, it still may be a good idea to provide the full list of capabilities up front instead of making it necessary to use PJL to make the inquiry.